EECS-4220-001		Final			Name_____________________________________________

100 pts total, 10 pts/ea

- In Siemens’ OB1, there is a FC1 accessed that does the following:

Build the function “MIX” to complete the operation.  Show all tables and logic inside the Function (FC1):

2. 	The following describes a function used by the Siemens PID block.  Describe how to accomplish the same using the A-B CompactLogix processor:

3. 	The following was an HMI diagram from a process.  Was this a well-developed or poorly developed HMI? Why? If not well developed, how would you improve it?

4.	Order the vessel level indicators from worst to best and give your rationale:

5.	For the following Modbus query, describe the slave’s response:

03

02

00

C6

00

11

CRC

6.	Use either the A-B or Siemens motion commands (either relative or absolute) to control the action of the conveyor and feeder shown below. Write a program to provide this function.

7.	An example was given in class describing how to control the pressure in a steel furnace even when the door was opened.  Describe of how you would accomplish this.  Be specific:

8.	The following conveyor system has five output lights for percent complete of packages going down conveyor 1 to conveyor 2.  Write a program to turn on these five output lights based on the fact that packages must pass photoeye 1 to enter the storage area and pass photoeye 2 to exit.

Describe how you could display the same data on an HMI panel using only one output device.  A-B is preferred.

# 9.	At the end of Ch. 19 is an article:

# A Discussion Comparing DCS and PLC/SCADA for Process Control

# DCS and PLC/SCADA – a comparison in use

The author stipulates:

### It may surprise you to know that PLC, HMI and SCADA implementations today are consistently proving more expensive than DCS for the same process or batch application. CEE finds out more…

What does the author claim for the basis of his arguments and what would you do as a PLC programmer to counter these claims?  Be specific:

10.	Using either the PID blocks from A-B or Siemens, provide a program that will work in auto mode for the following P&ID.  Use variables as inputs, outputs and internal variables as necessary.  Describe these variables in a table.  (Two pages are provided for this problem.)

Description of "02" Read Input Status

The "02" request reads the status of discrete input points.  The request is for inputs 197 to 218 from slave device 17.

Query

| Field Name |  |
| --- | --- |
| Slave Address | 11 |
| Function | 02 |
| Starting Address Hi | 00 |
| Starting Address Lo | C4 |
| No. of Points Hi | 00 |
| No. of Points Lo | 16 |
| Error Check (LRC or CRC) | ---- |

Response

| Field Name |  |
| --- | --- |
| Slave Address | 11 |
| Function | 02 |
| Byte Count | 03 |
| Data (Inputs 10204 - 10197) | AC |
| Data (Inputs 10212 - 10205) | DB |
| Data (Inputs 10218 - 10213) | 35 |
| Error Check (LRC or CRC) | ---- |

| 10204 | 10203 | 10202 | 10201 | 10200 | 10199 | 10198 | 10197 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 10212 | 10211 | 10210 | 10209 | 10208 | 10207 | 10206 | 10205 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 10220 | 10219 | 10218 | 10217 | 10216 | 10215 | 10214 | 10213 |
| 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |