EECS 4220	Quiz 9			Name_______________________________________________

- For the following Modbus query, describe the slave’s response:

03

02

00

C8

00

06

CRC

The "02" request reads the status of discrete input points.  The request is for inputs 197 to 218 from slave device 17.

Query

| Field Name |  |
| --- | --- |
| Slave Address | 11 |
| Function | 02 |
| Starting Address Hi | 00 |
| Starting Address Lo | C4 |
| No. of Points Hi | 00 |
| No. of Points Lo | 16 |
| Error Check (LRC or CRC) | ---- |

Response

| Field Name |  |
| --- | --- |
| Slave Address | 11 |
| Function | 02 |
| Byte Count | 03 |
| Data (Inputs 10204 - 10197) | AC |
| Data (Inputs 10212 - 10205) | DB |
| Data (Inputs 10218 - 10213) | 35 |
| Error Check (LRC or CRC) | ---- |

| 10204 | 10203 | 10202 | 10201 | 10200 | 10199 | 10198 | 10197 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 10212 | 10211 | 10210 | 10209 | 10208 | 10207 | 10206 | 10205 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 10220 | 10219 | 10218 | 10217 | 10216 | 10215 | 10214 | 10213 |
| 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |