EECS-4220-001		Final			Name_____________________________________________

100 pts total, 10 pts/ea

- In Siemens’ OB1, there is a FC1 accessed that does the following:

Build the function “SHIFT” to complete the operation.  Show all tables and logic inside the Function (FC):

2. 	For the following Modbus query, describe the slave’s response:

03

02

00

C6

00

0E

CRC

Using the example:

Description of "02" Read Input Status

The "02" request reads the status of discrete input points.  The request is for inputs 197 to 218 from slave device 17.

Query

| Field Name |  |
| --- | --- |
| Slave Address | 11 |
| Function | 02 |
| Starting Address Hi | 00 |
| Starting Address Lo | C4 |
| No. of Points Hi | 00 |
| No. of Points Lo | 16 |
| Error Check (LRC or CRC) | ---- |

Response

| Field Name |  |
| --- | --- |
| Slave Address | 11 |
| Function | 02 |
| Byte Count | 03 |
| Data (Inputs 10204 - 10197) | AC |
| Data (Inputs 10212 - 10205) | DB |
| Data (Inputs 10218 - 10213) | 35 |
| Error Check (LRC or CRC) | ---- |

| 10204 | 10203 | 10202 | 10201 | 10200 | 10199 | 10198 | 10197 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 10212 | 10211 | 10210 | 10209 | 10208 | 10207 | 10206 | 10205 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 10220 | 10219 | 10218 | 10217 | 10216 | 10215 | 10214 | 10213 |
| 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |

3.	Use the Siemens motion commands below to control the action of the two motors shown. Write a program to provide this function.  Add a start button to initiate the action.   Assume the drives has been previously reset, enabled, and homed.  After the action is complete, add a second button to return the wrapper to home position.  Assume the machine allows one complete wrap at the bottom and at the top.

4.	Write logic in Ladder to place the material in the festooner box.  The festooner moves back and forth right, then left for 50 turns or until 100# of material is added to the box.  Assume an arm motion that has two outputs – arm right and arm left to distribute the material.  Use the photo-eyes at the left and right to stop the arm and reverse the operation.  Give a short amount of time for the arm to come to rest after being sensed by the eye before moving in the reverse direction.  If the 100# limit is reached before 50 turns occur, the arm finishes its lap and comes to a rest at the left side near the start pb.  Provide an I/O Table and logic:

5.	Write a Whack-a-Mole game using ladder logic for two lights.  The game is to react to the light by pushing that particular button before the light turns off.  Construct the game so that the time between lights is stored in an array and the next light to turn on is also stored in an array.  Count the steps (light turn-ons) and stop at 30.  Count the number of successful hits and store in a location.

6.	Using either the PID blocks from A-B or Siemens, provide a program that will work in auto mode for the following P&ID.  Use variables as inputs, outputs and internal variables as necessary.  Describe these variables in a table.  PV’s and CV’s are marked in the diagram.  Unmarked SP’s are setpoints from the HMI screen.  Ignore sqrt operation.  Assume that FT’s give a flow rate and the sqrt function is performed at the transmitter, not the controller.

7.	If an analog input represents the output of a potentiometer and a command is given to turn an output from 0 to 360o, write a program that will give an alarm if the potentiometer does not see the shaft of the potentiometer turn to within 10% of the command’s position in 10 seconds.

8.	Identify each of the following Artificial Intelligence topics by their proper identifier:

(Tree Search, Bayes Formula, Fuzzy Logic, ARIMA, Neural Networks)

9.	Show by example how the following can be accomplished. Use programming statements and written description of HMI buttons/indicators.  Use A-B and Factory Talk to describe how the HMI is configured to have only one device the HMI screen.

10.	If you found a pulse was coming into the PLC at a faster rate than could be handled by the simple counting function, describe a method to solve the problem of counting these pulses accurately.

How does the Tank over Tank PLC control the speed of the bilge pump using a single dc output?

With the bicycle project, how does the PLC accurately read the voltage if it varies from 0-16 V but the analog input to the PLC has a range of 0-10 V?

With a scale control module, the update of weight only occurs each 1/3 second.  Can the system control the weight added more accurately than to the weight added in 1/3 second?  If yes, describe your method.