EECS 4220	Mid-Term Exam	           Name________________________________________________

- Write the DeMorgan of the circuit below using ladder format.

- Observe what happens when the buttons are pushed:

Fill in the table:

| ##### LT1 | PB 1 Off | PB1 On |
| --- | --- | --- |
| PB2 Off |  |  |
| PB2 On |  |  |

- A car wash with two bays has a pump supplying water pressure to the spray heads.  If a bay in use contact turns on, one pump automatically turns on.  If a water pressure switch is not satisfied (sufficient pressure) and a delay of 5 seconds occurs, a second pump turns on.  If, after the second pump turns on and water pressure is not satisfied and a 10 second delay occurs, a third pump is turned on.  All pumps stay on for the duration of the wash and turn off after both bay in use contacts turn off.

Definition of Inputs:

| Sensor | Function/State | Signal Assignment |
| --- | --- | --- |

Definition of Outputs:

| Actuator | Function/State | Signal Assignment |
| --- | --- | --- |

- Next, write the logic to control flow for the three pumps using ladder logic:

- Write the logic in Ladder to satisfy the following control problem:

The process depends on a level switch in the two bins at the bottom (Bin 1 and Bin 2).  For either bin to fill, it must be at a low level.  Then the conveyor C2 will turn on and Bin 1 or Bin 2 will call for material until the high level is met for the bin being filled.  The Screw Conveyor will run delivering material to the conveyor for a fill to either bin.  The direction of  Conveyor C2 must be correct as well (forward or reverse).  Also, Storage Bin 1 has a high and low level switch and will be filled from above by conveyor C1 as needed.

- What three documents are primarily used in determining the design of a control panel?  How do each contribute to the design?

- Convert the following to seal logic using Allen-Bradley Ladder Logic:

- What person is given credit as inventor of the modern PLC?

What company and location wrote the specification leading to the invention of the PLC:

- The following festooner lays material in a box by moving an arm back and forth.  The action reverses after a small time delay after hitting the end-of-travel photo-eye.  The box is considered complete when the weight exceeds 100 pounds or 50 back-and-forth actions – whichever occurs first.  Design an I/O table and write a program in Ladder Logic to perform the action.

- Use ladder logic to add a seventh timer if a person requests a pedestrian “all walk” segment of time for all lights to remain red while pedestrians can cross in any direction.  Include a request button and logic to add a seventh timer only if the request activated.  You do not need to add the output logic for the lights.