EECS 4220	Mid-Term Exam	           Name________________________________________________

- Write the DeMorgan of the circuit below using ladder format.

2.	For the Allen-Bradley processor below, draw the wires to connect a NO limit switch to input 14.

- A car wash with two bays has a pump supplying water pressure to the spray heads.  If a bay in use contact turns on, one pump automatically turns on.  If a water pressure switch is not satisfied (sufficient pressure) and a delay of 5 seconds occurs, a second pump turns on.  If, after the second pump turns on and water pressure is not satisfied and a 10 second delay occurs, a third pump is turned on.  All pumps stay on for the duration of the wash and turn off after both bay in use contacts turn off.

Definition of Inputs:

| Sensor | Function/State | Signal Assignment |
| --- | --- | --- |

Definition of Outputs:

| Actuator | Function/State | Signal Assignment |
| --- | --- | --- |

- Next, write the logic to control flow for the three pumps using ladder logic:

- The problem moves a box from conveyor 1 to conveyor 3.  Conveyor 2 acts as a grouping conveyor.  Four boxes are grouped on conveyor 2 before being sent to conveyor 3.  Fill in the  I/O table and write the ladder logic to move the boxes as described:

Definition of Inputs:

| Sensor | Function/State | Signal Assignment |
| --- | --- | --- |

Definition of Outputs:

| Actuator | Function/State | Signal Assignment |
| --- | --- | --- |

- Write logic in Ladder format to control the movement of the boxes to conveyor 3 where there is a reset button that signals that the boxes have been removed from conveyor 3 and boxes can continue to move from conveyor 1 to 2 to 3.

- What three documents are primarily used in determining the design of a control panel?  How do each contribute to the design?

- Convert the following to seal logic using Siemens Ladder Logic:

- Convert to Siemens Ladder code the following.  Define each tag by data type before using.  What is the function of this code?

- The following festooner lays material in a box by moving an arm back and forth.  The action reverses after a small time delay after hitting the end-of-travel photo-eye.  The box is considered complete when the weight exceeds 100 pounds or 50 back-and-forth actions – whichever occurs first.  Design an I/O table and write a program in Ladder Logic to perform the action.