EECS 4220	Quiz 5			Name_______________________________________________

Write a program to dump material from the box into a vat.  The process begins with a pushbutton  closure from an operator that directs the vibrator to start. Material then flows from the box.  The box continues to dump material until a setpoint is reached.  The weight of the box containing the material is recorded by the scale.  When the weight setpoint is reached, the vibrator stops and an output light  turns on for 60 seconds.  The control panel has the button to start the operation, a numeric output of the weight dumped and a target setpoint numeric entry.  Provide a list of inputs and outputs.  Use ladder format.