EECS 4220	Mid-Term Exam	           Name________________________________________________

- Write the DeMorgan of the circuit below using ladder format.

2.	For the Siemens 1200 processor below, draw the wires to connect a NO limit switch to input I0.4.

Also, draw the wires to connect a 24 VDC buzzer to output Q0.3.

- A car wash with two bays has a pump supplying water pressure to the spray heads.  If a bay in use contact turns on, one pump automatically turns on.  If a water pressure switch is not satisfied (sufficient pressure) and a delay of 5 seconds occurs, a second pump turns on.  If, after the second pump turns on and water pressure is not satisfied and a 10 second delay occurs, a third pump is turned on.  Pumps remain on until one of the bay in use contacts turns off at which time pump 2 and 3 turn off and the timer circuits again may turn on additional pumps if the pressure is not satisfied.  If both contacts turn off, all pumps turn off.

Fill in Definition of Inputs:

| Sensor | Function/State | Signal Assignment |
| --- | --- | --- |

Fill in Definition of Outputs:

| Actuator | Function/State | Signal Assignment |
| --- | --- | --- |

Next, write the ladder logic to control the three pumps:

- Convert the following seal circuit to an S/R circuit.

- Convert the following to Ladder Logic (Siemens S/R):

- Fill in the Siemens’ equivalent of the following A-B instruction:

- Fill in the equivalent Siemens of the following A-B instruction:

- From the following two figures below, first fill in the table of inputs and outputs:

Fill in Definition of Inputs:

| Sensor | Function/State | Signal Assignment |
| --- | --- | --- |

Fill in Definition of Outputs:

| Actuator | Function/State | Signal Assignment |
| --- | --- | --- |

- Then write logic to perform the automatic operation of the feeder:

- With the following liquid storage tank, the liquid comes into the tank from various sources and is pumped out when the high level (L1) is reached.  When the low level (L0) is reached in the pumping operation, a valve V1 opens momentarily to spray a cleanser in the tank followed by valve 2 which sprays water.  The timing diagram for these sprays is shown below. Write a program to control the pump and V1 and V2.  Spray times are left to the discretion of the programmer.