EECS 4220	Mid-Term Exam	           Name________________________________________________

10 problems – 10 pts/ea – 100 pts

- Write the DeMorgan of the circuit below using ladder format.

2	Convert the following seal circuit to a latch/unlatch circuit, to an S/R circuit.

- Lab 8.2B concludes with the following:

The experiment concludes with the values for the limits of QW1000 found. Completely clockwise is 175 and completely counter-cockwise is 1275.  The servo ranges from 0 to 180o in the process.

Your Task:  Write a program to move the gripper from 0 to 180o at a variable number of degrees per second.  Use the number of degrees per second as your variable that is input to the program.

- Write a program to place the material in the festooner box:

The material moves into the box with an arm that moves left and right. The sensors at each end stop the movement and, after a short time delay, reverse the motion.  The scale under the box records the weight at the beginning of the fill.  The arm stops close to when 100# is met.  The arm is to come to rest at the left sensor.  If the weight isn’t yet at 100# but closer to 100# than if the arm were to move through another pass, the arm is to stop and the operation is over.  The goal is to place as close to 100# material as possible with the fill ending at the left sensor.

- Write a program that turns on a heater when the temperature falls less than 300 oF and turns off the heater when the temperature exceeds 325 oF for the liquid in a vessel.  The temperature is input in a variable labelled ‘temp’.  In a table, define each variable used by type.

- Convert the following seal circuit to an S/R circuit.

- Write the logic in Ladder to satisfy the following control problem:

A box is placed on the first conveyor (C1).  Then the operator pulls the pull-cord and the conveyor starts if C2, the second conveyor, is also running.  If not, the conveyor C1 waits until C2 starts and then turns on.  The box moves on C1 until the trailing edge passes a photo-eye between the two conveyors.  Then C1 stops and waits for another box.  For this problem, the programmer does not control conveyor C2 but only has a contact from the conveyor C2 reporting its run status.

- Fill in the Siemens’ equivalent of the following A-B instruction:

- Fill in the equivalent Siemens of the following A-B instruction:

Name the three documents that give the legal instructions in the building of an electrical panel in a factory:

- With the following Allen-Bradley timer logic, there are problems with the logic to create 5 second timers for six successive timers for the traffic light problem.  Fix these problems and add logic to add a push button to allow pedestrians to walk in all directions for an interval of time while all lanes are red.