EECS 4220	Quiz 4			Name_______________________________________________

- Convert the following seal circuit to a S/R circuit.

- Write logic to place the material in the festooner box.  The festooner moves an arm back and forth placing the material in the box until 100 lbs is added to the box.  The arm moves until the photo-eye is reached.  The arm dwells for a second and then reverses direction. After 100 lbs is reached the festooner finishes placing this last pass and stops at the next photo-eye either at the left or right side of the box.