Quiz 5	EECS 4220				Name_________________________________________

The bike has two analog inputs, one for current and one for voltage.  Power is the instantaneous value V*I.  A timed interrupt program OB 30 is created which executes every 20 ms.  Write a program in OB 30 to calculate values to display for each of the following:

Volts

Amps

Power

Calories

Also, provide a reset button to zero the calories value.

The relationship of power to energy is:

Power(Watts) = energy(Joules) / time(seconds)

and

1 Joule = 0.238902957619 calories

Draw a state diagram for Automatic Mode for the following:

By setting switch S1 to “Automatic” and pressing “Start”, the automatic cycle is started.  A start may only occur if the feeder is in its homed position.  The automatic cycle ends in home position if the magazine is empty.

The automatic cycle stops after the actual step if:

1 – 	“Stop” button is pressed; automatic cycle resumes if “Start” is pressed again.

2 – 	If “Single Step” is activated.  Feeder steps forward one step in automatic cycle 			every time “Start” is pressed again.

The automatic cycle resets immediately if S1 is moved to any other operating mode.  The ongoing operation then stops immediately.  Starting automatic cycle again is only possible after the feeder has been brought into its homing position (either in manual mode or with the help of homing operation mode).

Interlocking Conditions which must be implemented first for security reasons:

Separator forth:	no part must be below gripper and NOT (x-axis in pick up position and gripper is 		not up)

Gripper down:		separator is back and x-axis in one of its end positions

X-Axis move left:	gripper is up and NOT (“x-axis move right” activated)

X-Axis move right:	gripper is up and NOT (“x-axis move left” activated)

Suction on:		--