EECS-4220-001		Final			Name_____________________________________________

100 pts total, 10 pts/ea

- In Siemens’ OB1, there is a FC1 accessed that does the following:

Build the function “INV” to complete the operation.  Show all tables and logic inside the Function (FC):

2. 	Using either A-B or Siemens ladder logic, write a cash register program that uses an accompanying HMI program with only three items [hamburger, fries, drink].  Include logic to calculate the total price ignoring tax.  Ignore the ‘cancel last’ button and combo logic as well.

3. 	Using A-B or Siemens ladder logic, write a program that could be used to show the movement of a bottle across a conveyor belt and populate a case.  Use a case of 12.  Use a start and stop button to start the operation.  Use a reset button to set the bottle counter in the case back to 0 and allow another operation to fill the case with bottles.

4.	The following is a section of the Simon-Says program in Ch. 13.  Rewrite these program statements using either A-B RSLogix 5000 or Siemens ladder code.

5.	For the process below, the roll conveyor carries boxes from left to right.  If a box is too close to the one before it, the spacing bar comes up (arrow) to hold it back until a constant spacing is achieved.  If a box is spaced in excess of the minimum, the box is allowed to pass on with no blocking.  Write a program in ladder logic to control the knife based on the photo-eye and an input from a pulse tachometer.  Assume the pulse tach is a dint word with a constantly increasing number of pulses each time read.

6.	Use the Siemens motion commands below to control the wrap of the pallet.  A button on the machine starts the wrapping and a light indicates that the operation is complete.  When the operation is complete, the operator pushes a second button which resets the shrink-wrap to the bottom to start again.

7.	Give an example of multiple inputs being used instead of just one value for the PV (Process Variable) of a PID Loop.  Write a program using either A-B or Siemens to demonstrate your answer.

8.	The following conveyor system has five output lights for percent complete of packages going down conveyor 1 to conveyor 2.  Write a program to turn on these five output lights based on the fact that packages must pass photoeye 1 to enter the storage area and pass photoeye 2 to exit.

9.	The following press operation depends on the operator pushing both buttons within 1/10 second of the other and holding them down during the press cycle.  While this operation now requires a safety function in order to operate the press, this logic was at one time allowed in a non-safety plc.  Write the logic to perform this operation in Ladder format.

Why wouldn’t the non-safety plc logic be allowed today?

10.	Using either the PID blocks from A-B or Siemens, provide a program that will work in auto mode for the following P&ID.  Use variables as inputs, outputs and internal variables as necessary.  Describe these variables in a table.  PV’s and CV’s are marked in the diagram.  Unmarked SP’s are setpoints from the HMI screen.  Ignore sqrt operation.  Assume that FT’s give a flow rate and the sqrt function is performed at the transmitter, not the controller.