EECS 4220	Mid-Term Exam	           Name________________________________________________

15 problems – 6 pt/ea  #16 10 pts, 100 pts total

- Write the DeMorgan of the circuit below using ladder format.

2.	Convert the following seal circuit to a latch/unlatch circuit, to an S/R circuit.

- Write a program to place the material in the festooner box:

The material moves into the box with an arm that moves left and right. The sensors at each end stop the movement and, after a short time delay, reverse the motion.  The operation is complete with 50 passes left to right and back left.  It is noticed that occasionally a photo-eye goes bad and the festooner continues to travel into the box causing a crash.  To stop the festooner, a timer is installed that allows travel in either direction only for 15 seconds.  If the photo-eye is not activated before 15 seconds, the festooner is to stop and sound an alarm.  Use Ladder logic to program this operation.

    - A low pressure switch is to be added to a fan starter circuit. Assign inputs for stop, start and low pressure.  If the fan loses pressure, then it is to shut down.  Design a start/stop seal circuit that controls the fan and stops on low pressure.  Use Ladder Logic.  Remember that when the fan is off, pressure is low.  Use the Ladder programming language.

- Convert the following seal circuit to an S/R circuit.

- Fill in the Siemens’ equivalent of the following A-B instruction:

- The following conveyor system has five outputs, lights for percent complete of packages going down conveyor 1 to conveyor 2.  Write a program using Ladder to turn on these lights based on the fact that packages must pass photoeye 1 to enter the storage area and pass photoeye 2 to exit.  Packages come into the temporary storage area and a person picks them up and manually places them on conveyor 2.

    - To start asynchronous wound motors, resistors are connected in the rotor circuit to avoid a high inrush current.  After pushing the start button S1, the line relay K1 is closed.  Then relays K2, K3 and K4 are closed in order, each after a time delay of 5 seconds from the previous relay closing.  Write the program to start M3 using Ladder.

    - Name the three documents that give the legal instructions in the building of an electrical panel in a factory:

    - A ceramic maker wants a certain piece of pottery to cook using the following profile.  Write a program in Ladder that outputs a setpoint to his controller after he pushes a start button.  At the end, turn on a light signaling that the process is complete.  A reset button turns the light off.  A hold toggle switch can be pushed at any time and the timer stops with the setpoint sticking at the present value. When the toggle switch is pushed again, the process continues from where it was when the toggle switch was first pushed.  A toggle switch works such that pushing it once closes the contact and pushing again opens the contact. The reset pushbutton can also be pushed prior to the complete light turning on.  This acts as a stop and ends the timing process and returns the setpoint to ambient temperature.  Remember Siemens Timers are TP, TON, TOF, TONR and A-B Timers are TON, TOF, RTO.  Increase and decrease the setpoint in 1 degree step increments.

11.	Use the description from the interviewer in Ch. 6 to draw a circuit to get the job:

“During the interview I simply asked, in writing, that the applicant draw a start/stop circuit ladder diagram using the following hardware: a normally open pushbutton, a normally closed pushbutton, a pilot light and a DPDT relay. I also noted the requirement was to turn on the green pilot light when the momentary start button was pressed and turn off the light when the momentary stop button was pressed. I also asked the applicant to add wire numbers, device designators and relay contact cross references.”

12.	For the Siemens 1200 processor below, draw the wires to connect a NO limit switch to input I0.5.

Also, draw the wires to connect a 24 VDC buzzer to output Q0.2.

- Write the ladder logic to turn on lights H1 and H2 to satisfy the following timing diagram.  By activating switch S1, the light H1 is switched on.  If S1 is activated again, a second light H2 becomes switched on.  By activating S1 the next time, both lights are switched off.  The pattern repeats…

- Give a best answer to each of the following:

- Name a PLC manufacturer that was absorbed by another PLC manufacturer.

- Name a PLC manufacturer that would be found predominantly in a Japanese auto company’s manufacturing facility.

- Name a PLC manufacturer that would be found predominantly in a European factory.

- Name a PLC manufacturer that would be found predominantly in a US factory:

- Write the ladder logic necessary to turn on the Change light if 45 cents is required for a candy bar and the rules similar to those of the lab are adhered to, that is, that 1, 2 or 3 coins can be ‘on’ when the request is pushed with the first is on before the second which is on before the third.  Be as complete as possible:

- Write the ladder logic to create six time intervals as in the traffic light problem but using only one timer: