Rockwell to Cognex AOP:  We have Cognex 7802 and 3805 cameras

https://www.cognex.com/videos/getting-started/rockwell-plc-cognex-add-on-profile-aop

With Siemens, install GSD files for Siemens

https://www.cognex.com/blogs/machine-vision/how-to-use-profinet-with-a-cognex-in-sight-camera

# CRX e-Learning

Learn the basics of setting up a CRX collaborative robot with our e-Learning training modules.

CRX e-Learning

- Overview and Features

- Unboxing and Setup

- Programming Part 1 – Initial Settings\

- Programming Part 2 – Manual Operation

- Teaching the CRX

- iRVision 2D Part 1 Setting up the Camera

iRVision 2D Part 2 Finding & Picking a Part

- iRVision 3D Part 1 Setting up the Camera

iRVision 3D Part 2 Finding & Picking a Partor

- Force Control Part 1 – Push Function

Force Control Part 2 – Phase Match

Force Control Part 3 – Centering Function

130 videos under:

https://techtransfer.fanucamerica.com/?categories=12881,8612

# Siemens to Fanuc Protocol Setup:

# EtherNet/IP Scanner – Connecting third party I/O’s using EtherNet/IP

- Entry

- Associated product(s)

The library used in this application example allows you to use EtherNet/IP field devices with SIMATIC controller.

Although, according to the number of installed nodes PROFINET is the largest Ethernet based fieldbus in the world, EtherNet/IP is at number 3. Especially in several regions of the world EtherNet/IP has a dominating role. SIMATIC controller inherently don’t have support for field devices using EtherNet/IP.

In this application example a possible solution is demonstrated and explained, how such EtherNet/IP field devices can be used with SIMATIC controllers.

As the Open User Communication allows a SIMATIC programmer to create the payload for packets send via UDP or TCP, the SIMATIC can implement the necessary features and functions to provide EtherNet/IP as loadable function block. This functionality can be used with any Open User Communication capable interface of the SIMATIC, such as integrated PROFINET ports as well as Industrial Ethernet CMs and CPs. Here SIMATIC controller applies to both S7-1500 and S7-1200.

The following graphic shows the configuration scenario the function was tested with.

The application example demonstrates all the necessary steps to configure the SIMATIC S7-1500 to operate as EtherNet/IP Scanner with the help of the LCCF_EnetScanner Library.

Downloads
  Documentation (1,9 MB)

Example project STEP 7 V19 (5,4 MB)
   Example project STEP 7 V18 (5,5 MB)
   Example project STEP 7 V17 (4,7 MB)
   Example project STEP 7 V16 (4,7 MB)
   Example project STEP 7 V15.1 (5,1 MB)

Library EtherNet/IP scanner STEP 7 V19 (1,9 MB)
   Library EtherNet/IP scanner STEP 7 V18 (1,9 MB)
   Library EtherNet/IP scanner STEP 7 V17 (1,9 MB)
   Library EtherNet/IP scanner STEP 7 V16 (1,9 MB)
   Library EtherNet/IP scanner STEP 7 V15.1 (1,9 MB)

Application example based on Point and Flex IOs

Downloads
  Documentation (2,5 MB)
   Example project Ethernet/IP IO scanner STEP 7 V18 (32,5 MB)
   Library Ethernet/IP IO scanner STEP 7 V18 (7,9 MB)

Further information
EtherNet/IP Adapter - connecting third-party control systems to SIMATIC S7 controllers via EtherNet/IP
CIP Server – providing process values via CIP with SIMATIC
CIP Client – exchanging process data with third party control systems

Complete setup:

https://www.youtube.com/watch?v=d5o3PmaFSU0

# FANUC Robot Ethernet Tutorial: Setting Up PLC I/O Communication

Learn how to set up Ethernet/IP communications, including steps for both the FANUC controller and the Studio 5000 environment for a Rockwell PLC.

Technical Article April 30, 2024 by Austin Futrell

Connecting a PLC to a FANUC robot via Ethernet/IP offers numerous benefits for industrial automation. It gives access to integrated control, allowing coordination between different manufacturing process components.

The PLC can send commands to the robot through standardized data exchange and receive real-time status updates, a feature that significantly enhances monitoring and control capabilities. Ethernet/IP's widespread use ensures easy integration with other devices on the factory floor, giving you confidence in the system's compatibility and scalability.

Required Controller Options for Ethernet/IP Communication

FANUC robots must be equipped with an Ethernet/IP communication option. If your standard robot installation doesn’t have this feature, you should contact FANUC for guidance and information on the process and costs of adding this feature to your robot system.

- Ethernet Global Data (EGD) IO (R793): This option allows your robot to exchange input/output data over Ethernet with other robots, eliminating the need for a PLC (Programmable Logic Controller). It simplifies connectivity by directly linking robots.

- Ethernet/IP Adapter (RTL-R784): With this setting, the robot acts as a slave device within a PLC-controlled network. It communicates using a specific IP address, allowing it to receive instructions and send information through the network.

- Ethernet/IP Scanner: Includes Adapter (RTL-R785): In this configuration, the robot serves as the master device on a network of slave devices. This allows the robot to control and coordinate the network activities.

- Ethernet Safety: Requires Adapter (RTL-R713): This feature enables the robot to safely exchange input/output with a safety PLC, like Rockwell GuardLogix or Compact GuardLogix. It allows safety signals to be managed over Ethernet/IP, reducing the need for traditional wiring safety measures in the work cell.

##### Figure 1. The location of the Ethernet Plug:1 within the R30iA/B controller. Image used courtesy of the author

### Setup Host Communication: FANUC Teach Pendant (R30iA/B Controller)

The robot's host communication settings must be configured to ensure proper communication through the Ethernet network.

##### Figure 2. Host Comm Setup: Menu > Setup > Host Comm > F3 for (Port), and then choose either port 1 or 2, depending on where you plugged in the Ethernet cable. Image used courtesy of the author

To set up the host communication, follow these steps:

- Navigate to the host comm setup page.

- Change the IP address for Port #1 or Port #2 as necessary. It's usual to set the IP address to almost match that of the PLC, differing only by the last number (octet) in the address.

- Click the right arrow in the bottom right corner to find the initialize button.

- Press the INIT button on the port to establish a connection.

- Under “Host Name (Local) Internet Address,” name your PLC and enter its IP address.

- Press the INIT button again while the “PLC Host” is highlighted. Again, initializing ensures everything is connected correctly.

- The “PING” feature can verify a successful connection.

- Cycle power through the teach pendant or robot controller to finalize the changes.

### Ethernet I/O Setup: FANUC Teach Pendant

In the Ethernet/IP adapter screen, you must specify the I/O size, measured in 16-bit word sizes. For example, if you require 32 bits for input and output, you would set the size to 2 words each. By default, the adapter connection is configured for four words (64 bits) each for inputs and outputs. Any changes to the I/O size will require you to restart the robot before the changes take effect.

Note: The I/O size set on the robot controller must match what is specified in the PLC device setup; this will be important when setting up the PLC Ethernet communications.

##### Figure 3. Ethernet/IP List Screen. ‘TRUE’ indicates that the PLC is enabled and operational. Image used courtesy of the author

##### Figure 4. Ethernet/IP Adapter screen. Image used courtesy of the author

The instructions for navigating to the Ethernet/IP adapter screen and setup are below.

- Press (MENU)

- Select (I/O)

- Choose (ETHERNET/IP)

- Press (ENTER)

- Use the cursor to navigate to the “ENABLE” column and turn on the adapter by pressing (TRUE) with an (F) command button. If you plan on changing the needed words, press (FALSE), modify the words, and then change it back to (TRUE).

- Select enter the desired connection number in the description column (the adapter connection we selected is [1]). Then, press (CONFIG).

- Enter your required input and output sizes.

- Lastly, restart the robot to apply the changes with [FCTN] > [CYCLE START].

### I/O Configuration: FANUC Teach Pendant

Using a FANUC teach pendant to configure I/O settings when setting up Ethernet/IP allows you to manage and monitor the I/O data the robot controller exchanges with a PLC. This includes enabling and configuring the I/O data, setting up communication parameters, and ensuring that the robot can effectively send and receive signals to control operations and respond to changes promptly. It’s about creating a communication pathway so the robot system integrates smoothly with the rest of the manufacturing or processing environment, through the PLC.

##### Figure 5. Navigate through Menu > I/O > Digital > Config to reach this screen. Image used courtesy of the author

Setting up the digital outputs is straightforward. Simply select the range of I/O you need. For example, if you need to use outputs 1-64, then map them accordingly within that range. The designated rack number for Ethernet/IP is “89”. With only one robot arm connected to the controller through a single adapter in slot one and aiming to access digital output 1, set the start position to 1.

Following the modifications, the STATUS of the adjusted outputs has shifted to PENDING. Restart the robot controller to activate these changes using the FCTN button.

UI and UO (UOP Signals) Configuration: FANUC Teach Pendant

UOP signals serve as the vital link for communication and coordination between FANUC robots and external devices like PLCs. These signals can provide functionalities like starting, stopping, and controlling robot programs directly through a PLC controller. These signals are configured similarly to digital outputs and serve specific functions assigned to each signal type. They are already labeled and tasked with specific roles.

To make a long story short, you will use these signals to start the robotic program from a PLC/ HMI. For future reference, [IMSTP], [HOLD], and [SFSPD] must be enabled to start a program.

Figure 6. Navigate through I/O > Type > UOP (IN/OUT) to reach these screens. Image used courtesy of the author

Enabling the I/O Signals

Now that the signals have been mapped and their functions understood, we need to activate them.

##### Figure 7. Enable UI signals: menu > system > config. Image used courtesy of the author

Figure 8. UOP auto assignment: menu > system > config. Image used courtesy of the author

##### Figure 9. Remote/local setup: menu > system > config. Image used courtesy of the author

First, we need to enable the UI signals to allow communication between the PLC and the robot through these signals. For setting #44, remember that the external signals (UI [1 to 18]) are turned off when the value is set to FALSE. This value should be switched to FULL to allow programs to be initiated by the PLC. Lastly, on line #42, change [LOCAL] to [REMOTE] to complete communication with the PLC.

PLC Communication: Studio 5000 Logix Designer

On the PLC side, the setup is fairly straightforward. We'll configure a generic Ethernet module via the “A1, Ethernet” tab. The accuracy of this setup is crucial, as any errors in data entry could prevent communication between the robot and the PLC.

Begin by entering the robot's correct IP address. Then, under connection parameters, ensure the assembly instance information matches; Input: 101, Output: 151, and Configuration: 100. The input and output can vary between different robotic controllers or different ports: see the robot manual for further instructions. The size should always correspond to the number of words you configured for your robot earlier in this tutorial (remember, we used 64 bits, which is 4x 16-bit words), and keep the configuration size at zero.

##### Figure 10. Locating the “Generic Ethernet Module.” Image used courtesy of the author

Figure 11. Configuration of the robot module Ethernet device. Ensure that the assembly instances are input correctly and that the sizes align with the settings previously input into your teach pendant. Image used courtesy of the author

### Checking Ethernet Communication Through I/O Monitoring

We will manually activate an output signal on the robot's teach pendant and then check if this change is reflected in the PLC for the robot's inputs. Remember, the robot’s digital output on the teach pendant is correlated to a robotic input signal on the PLC.

##### Figure 12. Digital output [45] is manually forced on for testing. Image used courtesy of  author

Figure 13. While the robot output is forced on, the PLC input shows successful communication. Image used courtesy of the author

This strategy works well for setting up a Logix PLC (Rockwell) to communicate with a robot. The exact steps for working with another PLC might differ, but the process of setting up the Ethernet assembly instances and sizes and determining the tags is often quite similar. Wit this setup, you should be able to handle all kinds of data exchange projects between the robot and the host PLC!

## Ethernet IP guide

### This is an Ethernet IP guide to help you when using this with Universal Robots

Last modified on Sep 30, 2024

NOTE: All files are avalable for download at the bottom of this page.

Examples are valid for:
CB3 Software version: 3.2.18654
e-Series Software version: All versions
Note that newer software versions may behave differently.

These examples can be used on CB3/CB3.1 from software 3.2 and e-Series.

Specifications

Here is an overview of some important information to get started.

PLC (used in example)

- Allen-Bradley Compact Logix L16ER

  - Studio 5000 Logix Designer
Note:

  - UR only supports Ethernet/IP Class 1 implicit or "I/O" messaging, not Ethernet/IP Class 3 explicit or "client/server" messaging.

  - Remember to change/select the correct CPU

Robot

- UR3, UR5, & UR 10 running Polyscope v. 3.2

Sample Programs:

The files listed are available for download at the bottom of this page and are used in the example (ACD project files are compatible with Logix Designer v.24):

- UR.ACD

- ur-eip-demo.urp

Conveyor Tracking example:

- EIP_conveyor_example.ACD

- UR-EIP-conveyor-tracking.urp

EDS file:

- UniversalRobot.eds

User-defined Data Types:

- UR_DataTypes.L5X

Note: The L5X file contains user-defined data types and can be used to import the message format into Rockwell Studio 5000 Logix Designer

I/O Message Format:

- eip-iomessage.pdf

Note: When using output registers, it is important to set the mask accordingly.

Script

Script Functions for synchronizing a program with a PLC:

- read_input_boolean_register(address)

- read_input_float_register(address)

- read_input_integer_register(address)

- read_output_boolean_register(address)

- read_output_float_register(address)

- read_output_integer_register(address)

- write_output_boolean_register(address, value)

- write_output_float_register(address, value)

- write_output_integer_register(address, value)

Notes

The lower range (bool[0:64], int[0:23], float[0:23]) of the gp input and output registers is reserved for FieldBus/PLC interface usage. The upper range (bool[64:127], int[23:47], float[23:47]) can be used by external RTDE clients (i.e URCAPS).

Trouble Shooting

- It is important to consider and set an RPI value that corresponds with the requirements of the installation. The valid range of RPI is 2ms - 3200ms where a smaller value means more frequent data synchronization but also requires more resources from the PLC, Robot and Network.

- If you experience connection issues between a scanner and the robot make sure that you have configured the scanner for unicast connection (point-to-point) and that you have sufficient network bandwidth between the scanner, the Robot and other devices to avoid colliding packets etc.

Setup

This guide gives step-by-step instruction on how to start using Ethernet/IP with Universal Robots or watch the video

Video: How to connect your UR robot to an Allen Bradley PLC

1. ROBOT: Enable Ethernet/IP in the Installation tab. Remember to save the installation afterwards for the changes to take effect the next time the installation is loaded.

2. ROBOT: The yellow LED indicates Ethernet/IP is running on the robot but no PLC/scanner is connected to the robot

3. PLC: Open UR.ACD project in Studio 5000 Logix Designer. If your version of Logix Designer is incompatible with the project file (i.e. older than v.24), you can create a new project and import the user-defined data types from the UR_DataTypes.L5X file.

4. PLC: Navigate to I/O Configuration -> Ethernet -> Ethernet-Module UR

5. PLC: Set IP Address of Robot and Press "Apply"

6. PLC: Navigate to "Connection" Tab and ensure that the RPI matches your requirements and that the check box next to "Use Unicast Connection over Ethernet/IP" is checked.

7. PLC: Download the program to the PLC

8. PLC: Confirm that "I/O OK" is solid green

9. ROBOT: Confirm that LED in Installation tab is also green

10. PLC: Navigate to Tasks -> Main Task -> Main Program -> Parameters and Local Tags

10.1. IF You did not use the demo, but will implement communication to your own code, you need to enable communication between the robot and the AB PLC. First set up URI and URO in your local or global variables, using the provided User Define Data Types:

10.2. Then setup CPS between the robot and the PLC

Ladder:

ST:

11. PLC: Navigate to URI -> URI.Robot inspect values sent from the robot

11.1. Example to test - Trend Velocity of XYZ

Demo

This demo is provided to show a basic example of how to communicate between the PLC and Robot. The demo allows the user to set two numbers in PLC output registers. The robot retrieves the numbers, calculate their sum and stores the result in a PLC input register.

1. ROBOT: Open ur-eip-demo.urp

2. ROBOT: Run program

3. ROBOT: Navigate to variables tab

4. PLC: Set integer registers URO.Int.Registers[0] and URO.Int.Registers[1] to some values

5. ROBOT: Go to the Variables tab and confirm that the numbers are received

6. PLC: Validate that the URI.Int.Registers[0] contains the sum

### Attached files

EIP_conveyor_example.ACD

UR.ACD

UR_DataTypes.L5X

UniversalRobot.eds

eip-iomessage.pdf

ur-eip-conveyor-tracking.urp

ur-eip-demo.urp

# Universal Robots announces seamless integration with Siemens PLCs

Company accelerates its commitment to “Automation for Anyone, Anywhere” by supporting the automation requirements of global companies.

InnovationNews

April 18, 2024   Universal Robots

## The SRCI is a new standard for robotics manufacturers

Universal Robots (UR), the world’s leading collaborative robot (cobot) company, has now integrated the Standard Robot Command Interface (SRCI) into its software. UR is proud to be among the first cobot vendors to offer this functionality.

The SRCI is a new standard for robotics manufacturers that aims to create a single interface between PLCs and robots. The full integration of SRCI into UR’s software will enhance the connectivity capabilities of UR’s cobots and will ensure customers a frictionless integration with Siemens PLCs (Programmable Logic Controllers), since Siemens is the first - and currently only - PLC Vendor supporting SRCI in the automation market.

“By having our robots connect seamlessly to a global industry leader like Siemens, we can now offer our partners and customers, both existing and prospective, faster integration and higher ease of use,“ says Global Director, Strategic Partnerships at Universal Robots, Daniel Friedman. “We strive to make cobot automation as simple as possible for our customers and this is yet another step in our commitment to provide automation for anyone, anywhere.”

The uniform cross-manufacturer data interface of the SRCI makes robot implementation interoperable, and it standardizes definitions and robot commands between UR cobots and Siemens PLCs. This allows for easier and quicker setup and simplifies the deployment of UR robots into existing and new Siemens ecosystem-based production lines.

“We are encouraged that cobots from Universal Robots can now be controlled via the SRCI and can be programmed by Simatic users,” says Rolf Heinsohn, Senior Vice President, Factory Automation Segment Control at Siemens. “We want to accelerate Factory Automation and scale the use of robots in industry by making them simple and available to all our customers` employees. It is a great advantage for our customers to be able to easily integrate and use UR’s robots together with the Siemens PLCs in their production.”

The SRCI is available for UR’s e-Series family and the next-generation robots UR20 and UR30. It can be installed and activated with PolyScope version 5.15 or higher via add-on URcap software. For more info, click here.

# Video tutorials

Here you can find step-by-step programming examples and tutorials for troubleshooting and repair tasks.

If you want to learn the essentials of programming a UR robot, check out the Free e-Learning section where you can find interactive e-learning for both e-Series and CB3 robots.

- Polyscope X

- URScript

- AI Accelerator

- Programming examples

- Safety

1 Introduction to the GUI of PolyScope X

In this module, you will see the brand new GUI of PolyScope X and where to find what. The next modul…

2 Configuration of a Tool using the wizard

See how to use the wizards to configure Payload, Center of Gravity (CoG), Tool Center Point (TCP) an…

3 How to make a simple Pick and Place Program

In this video we show how to make a simple Pick and Place program using PolyScope X.

4 Using Modules and functions to increase readability and reusability of your program

Learn how to make Modules and functions. This will increase the readability of your program, and mak…

5 Transform positions with Offsets to minimize the number of waypoints

Learn how to Transform a waypoint using a user-defined offset. This will minimize the number of wayp…

6 Make a Grid in your application

Learn how to use the Grid feature in PolyScope X, to minimize the number of waypoints needed. (It re…

7 A Simple CNC application using Grid, modules and functions, and Transforms

In this module the features: Modules and Functions, Grids, and Transform with offsets are used to bu…

8 Features of the Operator screen

Learn how to set up the operator screen. How to make input variable and selections accessible to the…

9 Short walkthrough of the CNC Lathe machine template

See a short walkthrough of the template for a CNC Lathe Machine application. Teach the key waypoints…

## PROFInet How-To Guide E-series

### This guide provides instructions for how to start using PROFINET with Universal Robots.

Last modified on Sep 30, 2024

NOTE: All files are available for download at the bottom of this page.

This guide is valid for: e-Series Software version 5.12
Note: older or newer software versions may behave differently.

### 1. setup

1. ROBOT: Tap the Installation tab and, under Fieldbus, select PROFINET. Tap Enable.

- Save the installation for the changes to take effect the next time the installation is loaded.

2. ROBOT: The yellow LED indicates PROFINET is running on the robot. No PLC/IO controller is connected to the robot.

3. PLC: Open a project in Siemens TIA Portal.

- In the Project Tree of the Project view, navigate to: Online access -> {your network adapter}.

- Click Update accessible devices.

4. PLC: Identify the robot and the PLC you want to connect. Assign IP addresses and names to the equipment.

- Expand the desired accessible device and double-click Online & diagnostics.

- In the device's diagnostics page, expand Functions and click Assign IP address.

- Enter the desired IP address and subnet mask, and tap Assign IP address.

- Still in the diagnostics page click Assign name.

- Set the PROFINET device name and click Assign name .

5. PLC: Import the Universal Robots' GSD file.

- Navigate to Options ->Manage general station description files (GSD).

- Select to the right GSD and install.

6. PLC: In the top of the project tree access Devices & networks.

- Locate the UR I/O device in the catalog path: Other field devices->PROFINET IO->I/O->Universal Robots A/S->Collaborative Robot->V1.0

- Drag-and-drop it into the network view and connect the PLC and the I/O device by dragging a line between their (green) PN/IE ports.

7. PLC: Double-click the I/O device in the Network view.

- All modules are pluggable, and are named according to their corresponding slot numbers.

- Each module can be drag-and-dropped from the Catalog->Module to the empty fields in the Device overview.

- In this example all modules are plugged in. Notice how the input addresses (I) and output addresses (Q) are assigned, since you will need these to access the data.

8. PLC: Double-click the I/O device in the Device view to get the module description.

- In the General tab select Ethernet addresses, and select Use IP protocol.

- Set the same IP address as those assigned to the online device.

- Deselect Generate PROFINET device name automatically and type in the same PROFINET device name as the one assigned to the online device.

9. PLC:
This step requires PLC firmware version 4.0 or higher.

- Navigate to External source files.

- In the project tree, double-click Add new external file to open the file  UR_datastruct.udt.

- Import the user-defined data types by right clicking the newly added external source file and select Generate blocks from source.

- In the PLC tags (or your own Data Block), create tags corresponding to the modules you have enabled. Make sure the input addresses (I) and output addresses (Q) of the tags match those of the modules.

If all modules are enabled and placed in order in the IO memory, create a single PLC input tag of the data type: UR_T2O and a single PLC output tag of the data type: UR_O2T to map all data.

Both are visible in PLC data types in the project tree.

10. PLC: Download the program to the PLC.

- In the menu bar click the download icon and tap Start search in the download dialog.

- Select the PLC and click Load.

11. PLC: Confirm all blocks, tags and modules in the project tree are green.

12. ROBOT: Confirm the LED in the Installation tab is green.

​

13. PLC: In the project tree, navigate to your tag table (or Data Block) and expand your input tag to inspect values sent from the robot.

2. Specifications

The following sections provide an overview of some important information to get started.

#### 2.0.1. Demo Sample Program

Sample files for this example are available for download at the bottom of this page:

- UR_profinet_demo.zap13: archived project file that can be loaded into the TIA Portal v13 (select Project->Retrieve)

- ur-pn-demo.urp: demo program for the robot

PLC (used in example)

- Siemens S7-1200, 1212C AC/DC/Rly

- Totally Integrated Automation Portal V13, STEP 7 Basic

Robot

- UR3, UR5 or UR 10 running PolyScope v. 3.3

#### 2.0.2. I/O Message Format

For details about how the IO format is distributed between the modules, see: pn-iomessage.pdf

The robot has 10 pluggable modules:

- Seven modules containing the data that can be read from the robot

- Three modules containing data that can be set on the robot

All modules are optional, but are fixed to a specific slot according to their names: e.g. the UR_3_T2O_Joints can be plugged into slot 3, contain data from the robot (T) to the PLC (O), namely joint measurements.

Note: When using output I/Os, it is important to set the mask accordingly.

User-defined Data Types

- For PLCs S7-1200 and S7-1500 (PLC Firmware v4.0 or higher): UR_datastruct.udt (can be imported into TIA Portal)

- For PLCs S7-200, S7-300 and S7-400: URI.AWL, URO.AWL (can be imported into SIMATIC and TIA Portal)

The UDT/AWL files contain user-defined data types and can be used to import the message format.

Note: If you import the UR data structure (the .udt file) and only plug in some of the modules, make sure to use the right user defined data types and match the input address (I) or output address (Q) of the module with the address of the data types in the tag table. This is also necessary if you drag the modules in "out of order", e.g. the module to slot 5 before the module to slot 2.

GSD file

GSDML-V2.42-UR-PROFIsafe-20220517.xml.zip. (Note that the folder name is PROFIsafe but some of the files are compatible with PROFInet)

#### 2.0.3. Script Functions

Script Functions for synchronizing a program with a PLC:

- read_input_boolean_register(address)

- read_input_float_register(address)

- read_input_integer_register(address)

- read_output_boolean_register(address)

- read_output_float_register(address)

- read_output_integer_register(address)

- write_output_boolean_register(address, value)

- write_output_float_register(address, value)

- write_output_integer_register(address, value)

#### 2.0.4. Notes

- If you change the IP of the robot in Network Settings while PROFINET is enabled, disable PROFINET and enable it again before the changes take effect.

- If the IP of the robot is changed using external PROFINET tools, e.g. through the TIA software, the changes will take effect immediately.

- The lower range (bool[0:63], int[0:23], float[0:23]) of the gp input and output registers is reserved for FieldBus/PLC interface usage. The upper range (bool[64:127], int[24:47], float[24:47]) can be used by external RTDE clients (i.e URCAPS).

### 3. Demo

This demo is provided to show a basic example of how to communicate between the PLC and the UR robot. The demo allows you to set two numbers in PLC output registers.

The robot retrieves the numbers, calculates their sum and stores the result in a PLC input register.

1. PLC: Navigate to the Main Program Block and set integer registers URO."Reg 1".Ints.Register[0] and URO."Reg 1".Ints.Register[1] to some values and download the program to the PLC

2. ROBOT: Open ur-pn-demo.urp and run the program. Navigate to the Variables tab and confirm the numbers are received.

4. Robot Modes

| Mode | Description |
| --- | --- |
| -1 | ROBOT_MODE_NO_CONTROLLER |
| 0 | ROBOT_MODE_DISCONNECTED |
| 1 | ROBOT_MODE_CONFIRM_SAFETY |
| 2 | ROBOT_MODE_BOOTING |
| 3 | ROBOT_MODE_POWER_OFF |
| 4 | ROBOT_MODE_POWER_ON |
| 5 | ROBOT_MODE_IDLE |
| 6 | ROBOT_MODE_BACKDRIVE |
| 7 | ROBOT_MODE_RUNNING |
| 8 | ROBOT_MODE_UPDATING_FIRMWARE |

5. Video Guides

Italian Original : Profinet connection between robot and PLC - YouTube

English Translated : Profinet connection between robot and PLC - YouTube

### 6. Example image of FailSafe Library

### Attached files

GSDML-0361-0001-UR.bmp

GSDML-V2.42-UR-PROFIsafe-20220517.xml.zip

IMMI_Euromap67_input_output_bits.pdf

URI.AWL

URO.AWL

UR_datastruct.udt

Universal_Robots_TIA_Library_FailSafe_Robot_Programs.zip

Universal_Robots_TIA_Library_FailSafe_V16_20231016_1159.zal16

Universal_Robots_TIA_Library_FailSafe_V17_20231016_1333.zal17

Universal_Robots_TIA_Library_FailSafe_V18_20231016_1502.zal18

pn-iomessage.pdf

ur-pn-demo.urp