# Setting up the Siemens S7-1200 PLC

This section describes how to set up the Siemens S7-1200 PLC in TIA Portal to connect with the In-Sight vision system.

Configuring the PLC Hardware

Importing the GSD File into the Catalog

Adding the In-Sight Vision System to the Hardware Configuration

Downloading the Configuration into the PLC

Creating Monitoring Variables

# Configuring the PLC Hardware

Configure the PLC hardware with TIA Portal by following the below steps:

- Launch TIA Portal, and create a new project.

- Add the S7-1200 PLC to the project. To do so, double-click on Add new device on the left pane of the user interface, then select the correct PLC type, as shown below.

- Double-click on Devices & networks on the left pane, select Network view on the top, then click on the PLC CPU symbol. Select General on the bottom and click on PROFINET interface. Set the IP address, subnet mask and gateway for the CPU as shown below.

# Importing the GSD File into the Catalog

The In-Sight vision system appears in the Catalog only if its GSD file is installed. This must be performed only once, when the vision system is configured on the PLC the first time.

To install the GSD file for the In-Sight vision system, follow the below steps:

- Click on the Options menu, and then on the Install general station description file (GSD) option.

- Browse the folder containing the GSD file:

  - In case of using an In-Sight D900 series vision system, the GSD file is located here:

C:\Program Files\Cognex\In-Sight\In-Sight Vision Suite Adapters\In-Sight ViDi 1.3.0\FFP\In-Sight D9xx\GSD

  - In case of using an In-Sight ViDi PC device, the GSD file is located here:

C:\Program Files\Cognex\In-Sight\In-Sight Vision Suite Adapters\In-Sight ViDi 1.3.0\FFP\In-Sight ViDi PC\GSD

- Tick the box in front of the GSDML file, then click on Install.

Adding the In-Sight Vision System to the Hardware Configuration

Double-click on Devices & networks on the left pane of TIA Portal, and select the Network view tab on the top of the screen. If you have performed the procedure of the Importing the GSD File into the Catalog topic, then the In-Sight vision system should appear in the Catalog on the right side of the window in the following location:

Other field devices => PROFINET IO => Controllers => Cognex Corp. => Cognex Vision Controllers => In-Sight Vision System

Once the vision system is found in the above location, perform the following steps to add it to the HW Config:

- Drag and drop vision system next to the PLC in the Network View.

- Establish a connection between the PLC and the vision system by connecting the small green boxes in the PLC and vision system symbols while holding down the left mouse button.

- Click on the vision system symbol, then select General on the bottom and click on PROFINET interface [X1]. Tick IP address is set directly at the device option and uncheck the box in front of Generate PROFINET device name automatically. Type the same name for the PROFINET device name that was configured for Station Name in In-Sight Vision Suite in the Setting up the In-Sight Vision System topic.

Note:

If the IP address set directly at the device option is checked (as shown below), the system keeps the IP address of the vision system. If it is unchecked, its IP address is changed to a value provided by the CPU.

# Downloading the Configuration into the PLC

Once the vision system has been added to the hardware configuration, download it to the PLC:

- Save the project, click on the PLC CPU symbol, and then on the Online. Select Download to device to start the download procedure.

- If the download is performed for the first time, then the Extended download to device window comes up. Select the PG/PC interface there. After that, click on the Start search button to show the available devices. Select the PLC from the list and click on the Load button.

- The program compiles the configuration. Once it is done, click on Load and then on the Finish button on the pop-up windows.

- After the download is finished, click on the Go online icon on the top of the window. If the PROFINET connection is working fine between the PLC and the vision system, then a green sign should be visible in the left top corner of the devices.

- If the connection is not established properly, then right-click the vision system symbol and select Assign device name from the context menu.

- Select the vision system from the list and click the Assign name button.

Once this step is done, the PROFINET connection is established properly.

# Creating Monitoring Variables

Once PROFINET communication has been set up properly, create the monitoring variables. To prepare for the procedure, first open the applicable PROFINET IO Module Reference Table (see this topic for In-Sight D900 series vision systems, and this topic for In-Sight ViDi PC devices). This reference table is used to check which signals are needed for the configured application. The necessary signals can be added to the symbols in TIA Portal, and can be named to monitor them easily.

Perform the following steps to create the monitoring variables:

- On the Project tree pane, go to the following location:

Vision System => PLC_1 [CPU 1211C DC/DC/DC] => PLC tags

- Double-click on Add new tag table. This adds Tag table_1: double-click on that to open it.

- Open the Device view tab on the top of the screen to check the Q and I addresses to use for each module.

- Open Tag table_1, and fill it according to the contents of the Device view tab , then save the project. This is necessary to see the names of the variables when they are monitored.

- On the Project tree pane, go to the following location:

Vision System => PLC_1 [CPU 1211C DC/DC/DC] => Watch and force tables

- Double-click on Add new watch table. This adds Watch table_1: double-click on that to open it.

- Fill the Address column as seen in the table below. Cognex recommends filling the Name column first, as the Address and Display format columns should be filled automatically when selected, according to the values configured in Step 4. Save these values for future use.

- Click the Monitor all icon, so that the Monitor value column is updated cyclically, and the changes coming from the vision system can be monitored. To modify a variable in this case, type the new value in the Modify value column, and then click the Modify all selected values once and now icon.

Note:

  - Analog values are indicated in hexadecimal format by default.

  - The System Ready bit should show TRUE if everything has been set up correctly.

# In-Sight D900 Vision System - PROFINET IO Module Reference Table

This topic provides the PROFINET IO Module reference table, which should be used in conjunction with the GSD file for incorporating the In-Sight vision system and a Siemens PLC via the PROFINET protocol.

Note: For detailed descriptions of the signals, see Industrial Communications Signals.

| Module | ID | Byte | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acquisition Control | 0x101 | 0 | Set Paused | Reserved |  |  |  | Clear Exposure Complete | Trigger | Trigger Enable |
| Acquisition Status | 0x201 | 0 | Online | Paused | Reserved |  | Acquisition Error | Exposure Complete | Trigger Acknowledge | Trigger Ready |
|  |  | 1 .. 2 | Acquisition ID |  |  |  |  |  |  |  |
| Inspection Control | 0x102 | 0 | Clear Error | Reserved |  |  | Execute Command | Set User Data | Inspection Results Acknowledge | Buffer Results Enable |
| Inspection Status | 0x203 | 0 | Set User Data Acknowledge | Command Failed | Command Completed | Command Executing | Results Valid | Results Buffer Overrun | Inspection Completed | System Ready |
|  |  | 1 | Error | Reserved |  |  |  |  |  | Results Passed |
|  |  | 2 .. 3 | Error ID |  |  |  |  |  |  |  |
| Command Control Input | 0x108 | 0 .. 1 | Command ID (16-bit) |  |  |  |  |  |  |  |
| Command Control Output | 0x108 | 0 .. 1 | Command Result Code (16-bit) |  |  |  |  |  |  |  |
|  |  | 2 .. 3 | Reserved (16-bit) |  |  |  |  |  |  |  |
| External Event Control Input | 0x106 | 0 | External Event 7 | External Event 6 | External Event 5 | External Event 4 | External Event 3 | External Event 2 | External Event 1 | External Event 0 |
| External Event Control Output | 0x106 | 0 | External Event Acknowledge 7 | External Event Acknowledge 6 | External Event Acknowledge 5 | External Event Acknowledge 4 | External Event Acknowledge 3 | External Event Acknowledge 2 | External Event Acknowledge 1 | External Event Acknowledge 0 |
| User Data | 0x301 (16) 0x302 (32) 0x302 (64) 0x304 (128) 0x305 (254) | 0 .. | User Data |  |  |  |  |  |  |  |
| Inspection Results | 0x401 (16) 0x402 (32) 0x403 (64) 0x404 (128) 0x405 (250) | 0 .. 1 | Inspection ID |  |  |  |  |  |  |  |
|  |  | 2 .. 3 | Inspection Result Code |  |  |  |  |  |  |  |
|  |  | 4 .. | Inspection Results |  |  |  |  |  |  |  |

Rockwell

# Establish a Unicast Implicit Connection - In-Sight 5.x.x Firmware

If the vision system is has In-Sight 5.x.x firmware version installed, the following steps must be taken to transfer data between an In-Sight vision system and a ControlLogix or CompactLogix PLC on an EtherNet/IP network, depending on the version of RSLogix 5000 being used to program the PLC.

## RSLogix 5000, Version 20+

- Install the EDS files included with In-Sight Explorer software. For more information, see Install the EDS Files.

- Establish the connection using the EDS generated profile. For more information, see Establish the Connection Using the EDS Generated Profile.

Install the EDS Files

Establish the Connection Using the EDS Generated Profile

Establish the Connection Using a Generic ETHERNET-MODULE

# Install the EDS Files

Electronic Data Sheets (EDS) are text files used by various network configuration tools, such as Rockwell Software's RSLinx™, to identify In-Sight vision systems on a network. These EDS files are then imported into the appropriate software.

This example will walk through integrating Cognex EDS files with Rockwell Software's RSLinx. To correctly identify In-Sight sensors within RSLinx, perform the following:

- Click Start > All Programs > Rockwell Software > RSLinx > Tools > EDS Hardware Installation Tool.

- Click Add.

- Select Register a directory of EDS files. Next, click the Browse button to navigate to the EDS subdirectory of the In-Sight Software installation directory. The default location is: C:\Program Files (x86)\Cognex\In-Sight\In-Sight Explorer 5.x.x\Factory Protocol Description\EDS. Click Next.

- Click Next.

- Click Next.

- Click Next.

- Click Finish.

- Click Exit to terminate the Hardware Installation Tool.

# Establish the Connection Using the EDS Generated Profile

In-Sight Explorer includes .L5X files, which can be used to copy PLC controller tag data to/from user-friendly program tags, representing signals in the In-Sight vision system running In-Sight 4.10.x or 5.x.x firmware.

Note:

- If using an EDS generated profile, although the maximum size of the Input Assembly is 500 bytes, the EDS generated profile will only allow a connection of up to 496 bytes.

- When using Rockwell RSLogix Studio 5000, version 21 through 25, the In-Sight EDS generated profile’s default Input and Output Assembly sizes (496 bytes for each) cannot be edited and will default to the largest Input and Output Assembly sizes.

- When using Rockwell RSLogix Studio 5000 version 19 and earlier, an Allen-Bradley Generic ETHERNET-MODULE must be used, instead of a Cognex Corporation In-Sight vision system specific module. For more information about adding and configuring a Generic ETHERNET-MODULE, see Establish the Connection Using a Generic ETHERNET-MODULE.

- Install the EDS files included with In-Sight Explorer software, if not already installed.

- Open RSLogix 5000 and load the PLC's project.

Note: The PLC must be Offline to add connections in RSLogix 5000.

- Add a connection to your Ethernet Communications Card. Under the I/O Configuration node, select the Ethernet Node under the Ethernet Module, right-click on the icon and select New Module from the menu.

- From the Select Module Type dialog, select the type of In-Sight vision system that will be connected to the PLC and click Create.

Note: After loading the new EDS files, new module types will show up and the appropriate module should be selected.

- From the New Module dialog, enter the In-Sight vision system's name and IP address, and then click the Change... button to configure the size of the connection.

- After adding the vision system to the RSLogix project, the L5X rung import file can be used to create structured data for the connection. Within the main program, right-click on an empty rung and select the Import Rungs... option. Navigate to the .L5X file and click the Import... button.

  - If using In-Sight 4.10.x firmware, import the InSight_12_CopyRung.L5X file, which is installed to the following location:

C:\Program Files (x86)\Cognex\In-Sight\In-Sight Explorer 5.x.x\Factory Protocol Description\RSLogix

  - If using In-Sight 5.x.x firmware, import the InSight_11_CopyRung.L5X file, which is installed to the following location:

C:\Program Files (x86)\Cognex\In-Sight\In-Sight Explorer 5.x.x\Factory Protocol Description\RSLogix

- In the Import Configuration dialog, select Tags and change the names of the created tags to match the configuration.

  - If using In-Sight 4.10.x firmware, set the InSight_12:I and InSight_12:O tags to the connection controller tags in the project so that they match the name given to the In-Sight vision system connection.

  - If using In-Sight 5.x.x firmware, set the InSight_11:I1 and InSight_11:O1 tags to the connection controller tags in the project so that they match the name given to the In-Sight vision system connection.

- Click the OK button of the Import Configuration dialog to close it. A rung will be added to the project, which includes the copy instructions for copying the data between the generic connection data and the structured data types included in the imported rung file.

When writing your ladder program, reference the elements in the structured data variables; the copy rung above will automatically move the data to and from the structured data instances to the EtherNet/IP connection. Download and run to get the tags to copy data initially, otherwise an array of SINTS will be all that is seen.