Lesson 1

Getting Started

Lesson 2

Getting Started – Teach Pendant

Lesson 3

Lesson 4 – Define Parts and Fixtures

Lesson 5

End of Arm Tool (EOAT)

Lesson 6

Teaching a Program

HandlingPro Simulation Programs and Editor:

Provides a limite but simple robot program editor.  This editor is used to quickly create programs that can animate workcells and validate cycle time.

HandlingPro Virtual Teach Pendant Editor:

HandlingPro provides a virtual Teach pendant that has a fully functioning teach pendant interface to the robot.  The teach pendant use the same software that executes ona physical robot.  The Virtual TP Editor is used to create full-instruction-set robot TP programs.

Program Generation:

Robot program can be generated from defined features in HandlingPro software.  Features are owned by the parts they are created on.  Since parts can be used in multiple fixtures, a feature can be used to generate programs for a part in several locations.

Program Grid – Displays a listing  of the selected program from which TP ines can be selected.

Show Motion Lines Only Checkbox – When checked the Program grid displays only motion ines contained in the selected TP program.

3D Select Multiple Points Button – When selected, the mouse can be used to select nodes.  To create a selection box, hold the left button at the start point and drag the mouse to the end point of the box.  TP nodes are highlighted as they are selected inside the box.

Position Adjustment Boxes – Boxes are shown for X, Y, Z, W, P, R. The value shown in the box is the amount selected positions will be bumped when an arrow button is pressed ().  Positions are bumped in the frame selected in the bump relative combo box.

Bump Relative – When selected a position is moved relative to the current position in the frame of reference selected in the bump reative combo box.

Create a Program from a Part using the Draw Feature

Condition:  CAD part is in the workcell

EOAT UTOOL value is adjusted appropriately

Lesson 7

Teaching a Program - Fixtures

Color: Defines the color used for the TP Trace.  To change the color select the Palette wheel and selector a color.

Coloring Style: The TP Trace can be colored based upon different program run characteristics:

Solid: a solid line is displayed for the TP Trace

Color by Speed: the TP Trace changes color based upon robot speed along the program.

Color by Orientation: the TP Trace changes color based upon Orientation changes along the

Program.

Color by Acceleration: the TP Trace changes color based upon the Acceleration change along the program.

Show Value Enable/Disable: Enable this option to display the values for the option selected in Coloring Style.

Keep Visible: If enabled the TP Trace stays visible in the workcell for the run selected.

Lesson 8 – Calibrating Objects

Lesson 9 - Machines

Lesson 10 – IO Interconnections

Lesson 11 – Importing/Exporting an Saving Files