Enabling the iPendant gives motion solely to whom?
The Controller

Assuming standard setup, if you jog the robot in World -Y, which direction will the TCP move?
To the robots right

What key is depressed to switch between the different jog modes?
COORD

What two buttons display the Jog Menu?
Shift and COORD

What type of memory is saved when performing an image backup?
F-ROM and SRAM

Singularity error can be corrected by moving? 
Joint 5 (+/-) 10 degrees

Where is the default Tool Center Point?
Center of faceplate

What type of I/O handles communication between the controller and the robot?
Robot I/O

Enabling the iPendant gives motion solely to whom?
The Controller

Assuming standard setup, if you jog the robot in World -Y, which direction will the TCP move?
To the robots right

What key is depressed to switch between the different jog modes?
COORD

What two buttons display the Jog Menu?

Shift and COORD

What type of memory is saved when performing an image backup?
F-ROM and SRAM

Singularity error can be corrected by moving?
Joint 5 (+/-) 10 degrees

Where is the default Tool Center Point?
Center of faceplate

What type of I/O handles communication between the controller and the robot?
Robot I/O

Positional data is dependent on what two frames?
User and Tool

Which type of instruction defines for the robot how to move, where to move, and how fast to move, and how to stop?
Motion instruction

Which type of pasting will paste the motion instructions in order and renumber the Position's ID's based on the first available Position ID?
POSITION

Which mode limits all cartesian speed to less than 250 mm/sec and joint speed to less than 10%?
T1 mode

Which motion type moves the TCP in a straight line from the start position to the destination position?
Linear

Which position must the DEADMAN switch be held to enable motion?
Centered

Before forcing an input it must be...
Simulated

What type of memory is saved when performing a file backup?
SRAM only

The six point method of defining a tool frame defines the location and ______ of a tool frame.
Orientation

What are the two possible actions of an IF/SELECT statement?
JMP LBL & CALL

The motion option OFFSET, PR[x] is offset relative to the orientation of what frame?
USER

Which macro assignments will execute a macro in a manual mode?
UK, SU, MF

How many soft keys are there on the iPendant?
5

What instruction will send program flow out to a sub-program?
CALL

If you would like to look at the positional data of P[3] in the instruction "J P[3] 100% FINE", what buttons would you press?
Cursor to the 3 of P[3] and press F5 POSITION

The motion option OFFSET, PR[x] is offset relative to the orientation of what frame?
USER

Which of the below is the number one safety concern in a robotic system?
people

When an Emergency Stop button is pushed
"dynamic" braking is employed on all joints and all brakes are engaged

The controller contains the computer that operates the robot. Which of the following is NOT housed within the controller?
Servo Motors

Which of these components shown in these pictures will allow high voltage to the robot?
Circuit Breaker

The jog speed is selected by pressing and releasing the:
+%/-% keys

Status indicator lights indicate the current status of the robot to an operator. These lights are:
to the left of the LCD screen on the teach pendant -- iPendant

If the HOLD status indicator is lit, which of the following does NOT occur?
all control digital outputs are turned off

Tool Center Point (TCP) can best be described as:
all of the above

Jogging the robot in the JOINT jog mode:
allows the operator to jog each joint individually, or a combination of joints by pressing more than one jog key simultaneously

Based on the guidelines for naming teach pendant programs (TPP), which of the following names would be considered acceptable?
PROG1

Given the following teach pendant program motion instruction, which of the following robot motions will occur?

L P[1] 1500mm/sec CNT75
a linear motion at 1500 millimeters per second to position P[1] with continuous motion termination and 75% corner rounding

Every point in a robot work envelope is definable based on either degrees of rotation from center, or zero, for the JOINT jog mode, or distance from the robot's origin for Cartesian motion. Positional data is viewed by accessing the:

position detail screen

Positional data in the WORLD jog mode consists of location, orientation and configuration. What is the orientation of a position?
The three real values relative to the rotation of the tool center point about the x, y, and z axes, referred to as yaw, pitch, and roll expressed in degrees.

While on the JOINT position detail screen, the current position is definable based on:
the degrees of rotation in either a positive or negative direction from zero

To modify the positional information of a taught position, jog the robot's tool center point (TCP) to the new position an press the:
SHIFT key and the corresponding function key for TOUCHUP

When needing to cursor along a line in a program to make changes to an element, pressing which option on the Function menu bar will display additional selection options for the element?
[CHOICE]

To select an existing program to work with, operators should press the __________ key.
SELECT

When step testing a program to verify the program's integrity:

A. the STEP status indicator should be lit
B. program statements may be tested independently in the forward (SHIFT+FWD) and backward (SHIFT+BWD) direction.
C. the DEADMAN switch must be gripped and help and the Teach Pendant enabled
D. all of the above

When continuously testing a program using the teach pendant:
single step testing must be disabled

Placing the robot in LOCAL mode when the Teach Pendant is disabled:
allows the operator to use the Cycle Start button on the SOP to start the currently selected program

Which teach pendant key do you press to abort a program?
FCTN

When jogging the robot, select the key on the Teach Pendant that must be pressed and held before you press a desired jog key.
SHIFT

To Cycle start a program from the Standard Operator Panel the REMOTE/LOCAL keyswitch is placed into the:
LOCAL position.

The "right-hand rule" in a Cartesian Coordinate System:

is helpful when determining the direction a robot will move while jogging in the WORLD.

What is the Teach Tool?
Allows you to "drag" the EOAT.

To add a Part to the workcell:
Select "Add a Part" from the Cell Browser within the Parts category.

To add End-of-Arm Tooling to the robot:
Select "UT:#(Eoat)" within the Tooling category of the Cell Browser.

To adjust the view of the robot use the zoom in and out command which is:
BOTH mouse (left and right) drag or use the mouse wheel.

To add a Pick Fixture to the Workcell:
From the Cell menu, select "Add a Fixture" or Cell Browser, right click on Fixtures

To remove a Fixture
All the above

To create a simulated program:
All the above.

To run and view a program from beginning to end utilizing the EOAT pick and place a part:
Click on Cycle Start within the icon bar.

To create an AVI file:
Click on the Record button within the icon bar.

For simulation to pickup and drop a part:
You must place a check in the "Allow part to be picked" and in the "Allow part to be placed" found within the Fixture Simulation tab.

How many jog frames are standard on the R-J3 series controller?
5

Select all methods that a Jog Frame can be taught.
3 point method
direct entry

What are the 3 methods used to define a tool frame.
3, 6, and direct entry method

What is the default location of the tool center point?
at the center of the faceplate

What are the three methods used to define a RTCP frame?
3 point, 4 point and direct entry method

How many user frames are standard on the R-J3 series controller?
9

The status line describes the current status of the I/O. When the status displays PENDING, this means:
the assignment is valid, but not active

Which I/O is used when using the EE connector on the robot?
robot

When group output binary is set to 7, what DO[#]'s will be switched on?
DO[1] = 1 DO[2] = 2 DO[3] = 4 D0[4] = 8 DO[5] = 16
1, 2, 3

An analog I/O signal is an input or output voltage that has a value within the range of the I/O board or module that is being used.
true

When inserting a program instruction, you must highlight either a line number or END. 
true

Timers must start and end in the same program.
false

When a CALL statement branches from the main program that executes the entire program, you must insert another CALL statement to return back to the main program.
false

Position register element instruction allows you to change an individual value in a joint or Cartesian position.
true

The following display is a direct or indirect addressing: R[3]=2
direct

The renumber function reassigns all position numbers in the program so that they are in sequential order. Renumbering changes the position ID, not the positional information.
true

will add lines exactly as they were copied and retains the current position numbers

F3 - POS ID

will add the lines exactly as they were copied, but leaves the position numbers blank

F2 – LOGIC

will add lines exactly as they were copied and renumbers the copied positions with the next available position number

F4 – POSITION

speed and termination types are pasted in the proper instructions to maintain the original speed and termination of each path segment

F3 -- RM-POS-ID

will add lines in reverse ore and does not record the positions, but leaves the position numbers blank

F1 -- R-LOGIC

this is exactly like the RM-POD-ID except that new position numbers are assigned in the pasted lines

F5 -- RM-POS

will add lines in reverse order and retains their original position numbers

F2 -- R-POS-ID

will add lines in reverse order and renumbers the copied positions with the next available position numbers

F4 -- R-POS

The replace option will help you to modify registers, motion speed and termination, inputs and outputs, jump labels, calls and time before and time after.

A. true
B. false

A macro that contains a motion instruction can be assigned to teach pendant user key 1 without the shift key.

A. true
B. false

A macro that contains a motion instruction can be assigned to user 1 button on the standard operator's panel when using R-J3iB and earlier controllers.

A. true
B. false

When assigning a number for the standard operator user key 1 for controllers R-J3iB and earlier, you can use SP[4].

A. true
B. false

When assigning user key SU[3}, the key that executes the macro is:

A. shift+tool1
B. shift+move menu
C. shift+I/O
D. shift+tool2

What start up method is used when turning on power to the robot and controller without entering boot monitor functions?

A. hot start
B. cold start

Can you always start the same program from the SOP cycle start while in remote mode?

A. yes
B. no

Each production check provides different enabled choices within the program select detail. Which condition takes priority when multiple conditions are enabled?

A. "check when run"
B. "check when resume"
C. "force condition"

When the controller is in local mode it cannot start from an external signal such as a PLC or cell controller.

A. true
B. false

When you enable the AT HOME check within the setup menu, the controller will check to see if the robot is at the home position and if it's not at home position then it will not run the production program.

A. true
B. false

You must turn the Teach Pendant ON in order to copy an existing program.

A. true
B. false

If you are saving files to a disk, you must set the default device. Which of the following menu items would you find this?

A. SETUP/DEVICE
B. FILE/UTILITIES

C. SELECT/UTILITIES
D. USER/SETTINGS

Teach Pendant programs can be loaded from the FILE menu and one other menu. What is the other screen?

A. SETUP menu
B. SELECT menu
C. UTILITIES menu
D. USER menu

To display the contents of a Device, you must Generate a Directory.

A. true
B. false

Turning the Teach Pendant on:

Gives motion control solely to the operator via the teach pendant

What must be accomplished to avoid hard-stops and over travels?

Set Axis Limits

SHIFT and the hard key COORD will display the following menu:

The "jog" menu

SHIFT and POINT are needed to do what?

Record the current position of the robot

In the motion command "J P [2] 100% FINE", If you wanted to change the "FINE" component to a "CNT100" what buttons would you press?

Select component, then press [CHOICE]

To switch manually from one program to another, you have to be in which screen?

Select

How many Robot I/O points are standard on a controller?

16

Which soft key is pressed to display the current motion instruction defaults from the EDIT Screen?

POINT

When creating a program, which hard key should be pressed?

Select

When using the Prog Adjust utility, which statement is correct?

Pressing CLR_ADJ or CLR_ALL will save the enabled data values

Which of the following tasks can be performed from the FILE Menu?

Get a default directory of the device

If while jogging the robot, you get an "in singularity" error, What must you do?

Joint jog J5 +/- at least 10 degrees

The "Program Header Information" at the DETAIL softkey enables you to modify the following program information, EXCEPT:

Active tool frame

What position is the suggested type of motion used to move the robot to a HOME position?

Joint

Which softkey allows you to enter additional program instructions into a teach pendant program?

[INST]

Which of the following is a type of I/O signal?

Digital I/O

Which of the following hard keys would bring the menu item ABORT ALL to the TP Screen?

FCTN

If you would like to look at the positional data of P[3] in the instruction "J P[3] 100% FINE", what buttons would you press?

Cursor to the 3 of P[3] and press F5 POSITION

What are the two menus from which you can save Teach Pendant files?

SELECT and FILE

Which is not a valid way to execute and assigned MACRO?

Holding PREV and NEXT

Which instruction will send a program flow out to a sub-program?

CALL

Which instruction will increment Register 7 (R[7]) by 1?

R[7]=R[7]+1

What are the two (2) possible actions of an IF/SELECT statement?

CALL and JMP/LBL

To Switch between JOINT, WORLD,TOOL,JGFRM, and USER for Jogging Purposes, you need to press which of the following keys?

COORD key

Where is Position Register (PR[ ]) data valid?

Globally through out the controller

Which of the following best describes the WORK ENVELOPE of the robotic arm?

The three dimensional space in which the robot end effector can move

When using the "Right Hand Rule", the Thumb represents:

The +/- Z Axis

The most important SAFETY concern in operating AUTOMATED equipment like industrial robots is:

The elimination of RISK

Where are the E-Stops Located?

The Teach Pendant and the Controller

Which of the following allows you to JOG in a true XYZ Cartesian Coordinate System?

WORLD

Which of the following modes can produce a SINGULARITY?

WORLD

How would you fix a DEADMAN Error?

Press the Deadman Switch halfway and then press Shift and Reset

What is the ORIGIN of the World Frame?

Centerline Intersection of J1 AND J2. AKA "MACHINE ZERO".

What is the Default Location of the Tool Center Point?

At the center of the faceplate.

What are the 3 methods used to define a tool frame?

3,6 point and direct entry

What button must be pressed prior to jogging the robot in joint mode?

SHIFT

Why is it useful to be able to define User Frames?

It allows an operator to set an origin point for a work cell, allowing it to be moved if needed.

Why is it useful to be able to redefine Tool Centerpoints?

It allows different EOAT to be attached to the same arm

Based on the guidelines for naming teach pendant programs (TPP), which of the following names would be considered acceptable?

PROG1

Which key combination is needed to access the split screen display menu?

Shift + Display

Which key is depressed to switch between different jog methods?

COORD

IF while jogging the robot you release the shift key, or jog key, what will happen?

Robot come to controlled stop

Three methods that can see current position

Joint, World, User

File Manipulation 7

What must be done prior to executing any type of backup or restore?

Set the default device/veryify the correct device is set

What key sequence is depressed and held to access the Controlled Start Screen while powering on the controller?

Prev and Next

What key sequence is depressed and held to access the BMON screen while powering the controller?

F1 and F5

What type of memory is saved when performing an Image Backup?

F-ROM and SRAM

What type of memory is saved when performing a File Backup?

SRAM only

When restoring a File Backup, SRAM files on the controller are over-written?

True

When restoring an Image Backup, F-ROM and SRAM files on the controller are erased/

True

What key is depressed to switch between the different jog methods?

COORD

What type of I/O handles communication between the controller and the robot?

Robot I/O

Positional data is dependent on what two frames?

User and Tool

Which type of instruction defines for the robot how to move, where to move, how fast to move, and how to stop?

Motion Instruction

Which type of pasting will paste the motion instruction in order and renumber the Position ID's based on first available Position ID?

Position

Before forcing an input it must be__________?

Simulated

What are the two possible actions of an IF/SELECT statement?

JMP LBL & CALL

The motion option OFFSET,PR[X] is offset relative to the orientation of what Frame?

USER

Which MACRO Assignment will execute a Macro in Manual Mode?

UK, SU, MF