## Cards – Robotics Final Study Set 100 Cards

| Term When backing up files to a memory device, all files are erased from the root directory. True or False. | Definition False. |
| --- | --- |
| Term When should a file backup be made? A. after major changes ## B. yearly C. after any changes D. monthly. | Definition C. after any changes. |
| Term The backed up images can be stored on only the same robot or on a robot with exactly the same memory configuration or larger. True or False. | Definition True. |
| Term Mode select key in the T1 position __ requires the robot to wait for a PLC command to start the current selected program __ is used when teaching the robot positions in space. In this mode, speed is limited to 250mm/s at 100% feedrate override __ is used when testing programs up to 2000mm/s at 100% of maximum robot speed __ is used when running full production. | Definition __ is used when teaching the robot positions in space. In this mode, speed is limited to 250mm/s at 100% feedrate override. |
| Term If you are saving files to a disk, you must set the default device. Which of the following menu items would you find this? A. setup/device B. select/utilities C. user/settings D. file/utilities. | Definition D. file/utilities. |
| Term [image] Identify each of the SOP components. | Definition 1. Fault reset 2. Cycle start 3. Fault light 4. Power light 5. Emergency stop. |
| Term When an Emergency Stop button is pushed __ dynamic braking is employed on all joints and all brakes are engaged __all digital I/O is turned off, then returned to the previous state when the program is resumed __ all digital I/O is turned off and the robot decelerates to a soft stop __Robot motion is stopped and the main power to the robot controller is disconnected. | Definition __ dynamic braking is employed on all joints and all brakes are engaged. |
| Term You must turn the Teach Pendant ON in order to copy an existing program. True or False. | Definition False. |
| Term When prioritizing the safety of your robotic system, the #1 consideration should always be: - Robot - tooling - people - work piece - external devices. | Definition People. |
| Term T1. | Definition Cartesian speed is less than 250 mm/sec and Joint speed is less than 10% of the maximum override speed. |
| Term Auto Mode. | Definition Robot operated at specified maximum speed. Cannot start programs using the Teach Pendant.. |
| Term T2. | Definition Full program speed, and override can be changed to 100%.. |
| Term Which backup type will backup F-ROM and S-RAM controller memory modules? A. File backup, then select all of the above B. Disk backup C. Image backup D. Hard drive backup. | Definition C. Image backup. |
| Term J1, J2, J3, J3, J5, J6 - Match each of the SOP components __ y axis - shoulder __ pitch __ z axis - elbow __ x axis - waist __ yaw - rotate about the z axis __ rotate. | Definition J2 - y axis - shoulder J5 - pitch J3 - z axis - elbow J1 - x axis - waist J6 - yaw - rotate about the z axis J4 - rotate. |
| Term When backing up an image what two keys are pressed on the Teach Pendant when powering up the controller to BMON? A. SHIFT FCTN B. SHIFT DATA C. PREV NEXT D. SHIFT MENU. | Definition C. PREV NEXT. |
| Term To display the contents of a Device, you must Generate a Directory? True or False. | Definition True. |
| Term Which of these components will allow high voltage to the robot? __ Teach pendant on switch __ ON button __ Circuit Breaker __ PLC. | Definition Circuit Breaker. |
| Term The controller contains the computer that operates the robot. Which of the following is Not housed within the controller? __ Control circuitry __ Operator controls __ Memory __ Servo motor. | Definition Servo Motor. |
| Term Teach Pendant programs can be loaded from the FILE menu and one other menu. What is the other screen? A. SELECT MENU B. SETUP MENU C. USER MENU D. UTILITIES MENU. | Definition A. SELECT MENU. |
| Term The Teach Pendant must be on in order to access the menus? True or False. | Definition False. |
| Term If the HOLD status indicator is lit, which of the following does NOT occur? A. the program has been paused B. smooth deceleration has occurred C. power is maintained on the robot and execution can be resumed D. all control digital outputs are turned off. | Definition D. all control digital outputs are turned off. |
| Term The R-JiB and higher iPendant is able to have multiple windows and provide Internet/Intranet access? True or False. | Definition True. |
| Term What is different between an R-30iB Teach Pendant jog keys and R-30iA. A. faster B. slower C. back light D. 2 additional jog keys. | Definition D. 2 additional jog keys. |
| Term The jog speed is selected by pressing and releasing the: A. FCTN key B. +%/-% keys C. arrow keys D. COORD keys. | Definition B. +%/-% keys. |
| Term Status indicator lights indicate the current status of the robot to an operator. These lights are: A. to the left of the LCD screen on the teach pendant -- iPendant B. at the base of the robot C. in the center of the teach pendants LCD screen display D. on the Standard Operators Panel (SOP). | Definition A. to the left of the LCD screen on the teach pendant -- iPendant. |
| Term Jogging the robot in the JOINT jog mode: A. moves the robot in a linear motion where precision is desirable B. moves the robot based on a Cartesian (XYZ) coordinate system C. allows the operator to jog each joint individually, or a combination of joints by pressing more than one jog key simultaneously D. prevents override speed from exceeding 15%. | Definition C. allows the operator to jog each joint individually, or a combination of joints by pressing more than one jog key simultaneously. |
| Term Tool Center Point (TCP) can best be described as: A. the point at the center of the robot's faceplate on a newly purchased robot B. the location on the end of arm tooling where work will be performed. C. the point on the tool that is used to teach positions D. all of the above. | Definition D. all of the above. |
| Term What key do you press to change Coordinates to jog in JOINT mode? A. COORD B. SHIFT C. EDIT D. SELECT E. MENUS. | Definition A. COORD. |
| Term To select an existing program to work with, operator should press the ______ key? A. MENUS B. NEXT C. SETUP D. SELECT. | Definition D. SELECT. |
| Term Quick/full menus can be accessed from the FCTN key? True or False. | Definition True. |
| Term What key can be depressed to access the Position screen? A. POSN B. MENUS C. SELECT D. DATA. | Definition A. POSN. |
| Term Which one is not a jog speed type? A. COURSE B. FINE C. VFINE D. 50%. | Definition A. COURSE. |
| Term Using a 3 position Deadman switch, which position allows you to jog the robot? A. 1 B. 2 C. 3. | Definition B. 2. |
| Term Name two ways to control the robots motion limits/boundaries. A. DCS and POSN limits B. DCS and AXIS limits C. POSN limits and AXIS limits D. POSN limits and Mechanical stops. | Definition B. DCS and AXIS limits. |
| Term Robot ___________ can be useful for getting help and ordering parts. A. Serial Number B. F Number C. Manufacturing Number D. Model Number. | Definition B. F Number. |
| Term Singularity error can be corrected by moving? A. Joint 5 (+/-) 10 degrees B. Joint 1 (+/-) 10 degrees C. Joint 2 (+/-) 10 degrees D. Joint 3 (+/-) 10 degrees E. Joint 6 (+/-) 10 degrees F. Joint 4 (+/-) 10 degrees. | Definition A. Joint 5 (+/-) 10 degrees. |
| Term The six point method defines the locaton and ____ of the TOOL FRAME? A. rotation B. distance C. position D. orientation. | Definition D. orientation. |
| Term What two buttons are pressed to display the JOG menu? A. SHIFT + MENU B. SHIFT + FCTN C. SHIFT + JOG D. SHIFT = COORD. | Definition D. SHIFT = COORD. |
| Term Name the three methods of defining a USER FRAME? A. five point B. position entry C. direct entry D. three point E. four point F. six point. | Definition C. direct entry D. three point E. four point. |
| Term What are the three methods used to define a RTCP frame? A. 3 point, 4 point, and direct entry method B. 3 & 7 point methods C. 2 and 6 point methods D. all of the above. | Definition A. 3 point, 4 point, and direct entry method. |
| Term If you are jogging in JOINT, the axis number indication the axis (or axes) in an over travel will be displayed in the error log. You can manually release over travel on your system from the ___________ Release screen. A. AXIS LIMITS B. POSN C. OT D. FCTN. | Definition C. OT. |
| Term How many User Frames are standard on the R-J3 series controller? A. 8 B. 11 C. 9 D. 13. | Definition C. 9. |
| Term What are the 3 methods used to define a tool frame. A. 4, 3, and six point method B. 3, 6, and direct entry method C. 2, 4, and 6 point method D. 2, 3, and direct entry. | Definition B. 3, 6, and direct entry method. |
| Term Select all methods that a Jog Frame can be taught. A. 3 point method B. 6 point method C. 2 point method D. direct entry E. 7 point method. | Definition A. 3 point method D. direct entry. |
| Term What is the default TOOL center point A. Face plate B. Set by TOOL frame C. World origin 0,0,0 D. End of TOOL. | Definition A. Face plate. |
| Term If you had to place an object at a 45 degree angle, what cooordinate would assist in accomplishing jogging the robot along this angle? A. USER FRAME B. WORLD FRAME C. TOOL FRAME D. JOINT FRAME. | Definition A. USER FRAME. |
| Term When a DCS error occurs, what two keys do you press in order to clear the fault? A. SHIFT RESET B. MENU RESET C. FCTN RESET D. FAULT RESET. | Definition A. SHIFT RESET. |
| Term What is the default location of the Tool Center Point? A. At the center of the robot B. the jog frame C. the center of the user frame D. the center of the faceplate. | Definition D. the center of the faceplate. |
| Term How many Jog Frames are standard on the R-J3 series controller? A. 5 B. 3 C. 2 D. 4. | Definition A. 5. |
| Term What key combination is used to record a program point? A. SHIFT + POINT B. SHIFT + SELECT C. SHIFT + PROGRAM D. SHIFT + RECORD. | Definition A. SHIFT + POINT. |
| Term While on the JOINT position detail screen, the current position is definable based on: A. the number of joints moved simultaneously B. the degrees of rotation in either a positive or negative direction from zero C. default values appearing on the position detail screen D. the Cartesian coordinate system. | Definition B. the degrees of rotation in either a positive or negative direction from zero. |
| Term Identify the motion instruction elements 1. J 2. P[1] 3. 100% 4. FINE 5. OFFSET, PR[2] ___ SPEED ___ TERMINATION ___POSITION ___TYPE ___OPTIONS. | Definition 100% - SPEED FINE - TERMINATION P[1] - POSITION J - TYPE OFFSET, PR[2] - TYPE. |
| Term Given the following teach pendant program motion instruction, which of the following robot motions will occur? L P[1] 1500mm/sec CNT75 A. joint motion at 50% override to position P[1] with fine motion termination B. a linear motion at 50% override to position P[1] with fine motion termination C. a linear motion at 1500 millimeters per second to position P[1] with continuous motion termination and 75% corner rounding D. a joint motion at 75% override to position P[1] with fine motion termination. | Definition C. a linear motion at 1500 millimeters per second to position P[1] with continuous motion termination and 75% corner rounding. |
| Term When needing to cursor along a line in a program to make changes to an element, pressing which option on the Function menu bar will display additional selection options for the element? A. LIST B. POSITION C. [CHOICE] D. EDIT. | Definition C. [CHOICE]. |
| Term Within the program header, what does Group Mask represent when it displays the following? [1,*,*,*,*] A. 1 MOTION GROUP B. 1 USER FRAME C. 1 TOOL FRAME D. 1 JOG FRAME. | Definition A. 1 MOTION GROUP. |
| Term What keys are pressed to change the default motion? A. MENU MOTION B. POINT EDDEF C. MENU POINT D. DATA MOTION. | Definition B. POINT EDDEF. |
| Term To modify the positional information of a taught position, jog the robot's tool center point (TCP) to the new position an press the: A. SAVE key in the FUNCTIONS menu B. SHIFT key and the corresponding function key for TOUCHUP C. INST key from the TPP teach pendant D. DEADMAN switch, enable the teach pendant, then press the RECORD pushbutton on the Standard Operator Panel. | Definition B. SHIFT key and the corresponding function key for TOUCHUP. |
| Term What combination of keys are used to record a position? A. SHIFT + RECORD B. SHIFT + PROGRAM C. SHIFT + SELECT D. SHIFT + POINT. | Definition D. SHIFT + POINT. |
| Term To use the cycle start button from the SOP, which of the following setup modes should the Remote/Local be set within the System/Config screen? A. EXTERNAL I/O B. SOP C. REMOTE D. LOCAL. | Definition D. LOCAL. |
| Term A Motion Group can define different groups of ________ that can be driven by FANUC servo motors used for independent pieces of equipment? A. axis B. arms C. tools D. frames. | Definition A. axis. |
| Term Where is Position Register (PR[ ]) data valid? A. GLOBAL B. LOCAL C. WORLD D. PROGRAM. | Definition A. GLOBAL. |
| Term What symbol is used to indicate that the robot is at the taught position? A. * B. + C. # D. @. | Definition D. @. |
| Term What key combination is used to touchup a program point? A. SHIFT + RECORD B. SHIFT + TOUCHUP C. SHIFT + SELECT D. SHIFT + PROGRAM. | Definition B. SHIFT + TOUCHUP. |
| Term Based on the guidelines for naming teach pendant programs (TPP), which of the following names would be considered acceptable? A. PROG#1 B. 1PROG C. PROG1 D. PROG 1. | Definition C. PROG1. |
| Term Positional data in the WORLD jog mode consists of location, orientation and configuration. What is the orientation of a position? A. the three values relative to the position of the x, y, and z axes and their distance from the zero position or origin measured in mm B. The three real values relative to the rotation of the tool center point about the x, y, and z axes, referred to as yaw, pitch, and roll expressed in degrees. C. the data structure consisting of values that reference the unique set of joint angles that are required to reach a point in space where there is more than one possible way to get there D. all of the above. | Definition B. The three real values relative to the rotation of the tool center point about the x, y, and z axes, referred to as yaw, pitch, and roll expressed in degrees.. |
| Term Every point in a robot work envelope is definable based on either degrees of rotation from center, or zero, for the JOINT jog mode, or distance from the robot's origin for Cartesian motion. Positional data is viewed by accessing the: A. status screen B. DATA key C. main menu D. position detail screen. | Definition D. position detail screen. |
| Term What status indicator will turn green when the robot step mode is off? A. CONTINUOUS B. STEP C. RUN D. LINE. | Definition B. STEP. |
| Term FINE termination type causes the robot to _____ at the destination position before moving to the next position. A. MOVE CONTINUOUSLY B. SPEED UP C. STOP D. SLOW DOWN. | Definition C. STOP. |
| Term What does RM-POS-ID do? A. pastes the lines in revers order and leaves the position numbers blank B. pastes the lines in reverse order and retains the current position numbers C. pastes the lines in reverse order and renumbers the positions D. paste all instructions except motion instructions in reverse order. | Definition D. paste all instructions except motion instructions in reverse order. |
| Term Which instruction will increment Register 7 (R[7]) by 1? A. R[7] ++ B. R[7] = 1 C. R[7] = R[7] + 1 D. R[7] + 1. | Definition C. R[7] = R[7] + 1. |
| Term What are the two possible actions of an IF/SELECT statement? A. SELECT & SET AN OUPUT B. JMP/LBL & SELECT C. SET AN OUTPUT & CALL D. CALL & JMP/LBL. | Definition D. CALL & JMP/LBL. |
| Term Match the Paste Reverse to the appropriate definitions. 1. F1 - R-LOGIC 2. F2 - R-POS-ID 3. F3 - RM - POS-ID 4. F4 - R-POS 5. F5 - RM-POS ___Will add lines in reverse order and retains their original position numbers. ___This is exactly like the RM-POS-ID except that new Postion numbers are assigned in the pasted lines. ___ Speed and termination types are pasted in the proper instructions to maintain the original speed and termination of each path segment ___Will add lines in reverse order and renumbers the copied positions with the next available position numbers. ___Will add lines in reverse order and does not record the positions, but leaves the position numbers blank.. | Definition _2_Will add lines in reverse order and retains their original position numbers. _5_This is exactly like the RM-POS-ID except that new Postion numbers are assigned in the pasted lines. _3_ Speed and termination types are pasted in the proper instructions to maintain the original speed and termination of each path segment _4_Will add lines in reverse order and renumbers the copied positions with the next available position numbers. _1_Will add lines in reverse order and does not record the positions, but leaves the position numbers blank.. |
| Term What feature from the EDCMD would you use to replace a CNT termination type with a FINE termination type? A. FINE B. SELECT C. REPLACE D. TYPE. | Definition C. REPLACE. |
| Term The following display is a Direct or Indirect addressing? R[3}=2 A. DIRECT B. INDIRECT. | Definition A. DIRECT. |
| Term Which option would you use to paste instructions that would include reverse positional data and new number ID? A. R POS B. R POS ID C. RM D. RM ID. | Definition A. R POS. |
| Term Match to the appropriate definition 1. F2 - LOGIC 2. F3 - POS ID 3. F4 - POSITION ___will add lines exactly as they were copied and renumbers the copied positions with the next available position number ___will add lines exactly as they were copied and retains the current position numbers ___will add the lines exactly as they were copied, but leaves the position numbers blank. | Definition _3_will add lines exactly as they were copied and renumbers the copied positions with the next available position number _2_will add lines exactly as they were copied and retains the current position numbers _1_will add the lines exactly as they were copied, but leaves the position numbers blank. |
| Term The RENUMBER function reassigns all position numbers in the program so that they are in sequential order. RENUMBERING change the POSITION ID NOT THE POSITION INFORMATION? TRUE OR FALSE. | Definition TRUE. |
| Term What instruction will send program flow out to a sub-program? A. JMP/LBL B. REMARK C. SELECT D. CALL. | Definition D. CALL. |
| Term When inserting a Program Instruction, you must highlight either a line number or END? TRUE OR FALSE. | Definition TRUE. |
| Term When a CALL statement branches from the Main program that executes the entire program, you must insert another Call statement to return back to the Main program TRUE OR FALSE. | Definition FALSE. |
| Term The REPLACE option will help you to Modify Registers, Motion Speed and Termination, Inputs and Outputs, Jump Labels, Calls and Time Before and Time After? TRUE OR FALSE. | Definition TRUE. |
| Term Timers must start and end in the same program? TRUE OR FALSE. | Definition FALSE. |
| Term Match the options to the appropriate definition 1. LOGIC 2. POS ID 3. POSITION ___Will add lines exactly as they were copied and renumbers the copied position with the next available position number ___Will add the lines exactly as they were copied, but leave the position numbers blank ___Retains the current position numbers. | Definition _3_ Will add lines exactly as they were copied and renumbers the copied position with the next available position number _1_ Will add the lines exactly as they were copied, but leave the position numbers blank _2_ Retains the current position numbers. |
| Term Position Register element instruction allows you to change an individual value in a Joint or Cartesian position? TRUE OR FALSE. | Definition TRUE. |
| Term Which of the following is a type of I/O signal? A. UIP I/O B. SIP I/O C. DIGITAL I/O D. CONFIGURATION I/O. | Definition C. DIGITAL I/O. |
| Term [image] A Macro that contains a motion instruction can be assigned to User Key 1 button on the Standard Operator Panel when using R-J3iB and earlier controllers? TRUE OR FALSE. | Definition TRUE. |
| Term The following Position Register Instructions will alter what coordinate? PR[1,3:LPOS]=1000 A. Y B. ALL C. Z D. X. | Definition C. Z. |
| Term [image] When assigning a number for the Standard operator User Key 1 for controllers R-J3iB and earlier, you can only use SP[4]? TRUE OR FALSE. | Definition TRUE. |
| Term Which is not a valid way to execute an assigned MACRO? A. Executing an item from the Manual Functions screen B. Pressing "USER 1" button on the SOP for earlier controllers C. Pressing a Teach Pendant USER key D. Holding PREV and NEXT keys. | Definition D. Holding PREV and NEXT keys. |
| Term [image] When Group Ouptput binary is set to 7 what DO[#]'s will be switched ON? A. 1,2,3 B. 3,4 C. 2,5 D. 2,3,4. | Definition A. 1,2,3. |
| Term The status line describes the current status of the I/O. When the status displays PENDING, this means: A. an assignment has not been made B. the assignment is valid, but not active C. the assignment is valid and active D. the assignment is invalid based on the I/O hardware present when the controller was turned ON. | Definition B. the assignment is valid, but not active. |
| Term An analog I/O signal is an input or output voltage that has a value within the range of the I/O board or module that is being used? TRUE OR FALSE. | Definition TRUE. |
| Term The Remark Miscellaneous Instruction allows what? A. none of the listed options B. markings C. data D. comments. | Definition D. comments. |
| Term Where is Position Register (PR[ ]) data valid? A. Nowhere, Position registers are an instruction and therefore do not have data B. Globally through the controller C. Within a maximum of 2 programs D. Within the current program only. | Definition B. Globally through the controller. |
| Term [image] When assigning User Key SU[3}, the key that executes the macro is? A. Shift + Move Menu B. Shift + I/O C. Shift + Tool1 D. Shift + Tool2. | Definition A. Shift + Move Menu. |
| Term [image] Configuring I/O establishes the correspondence between the signal number and the physical port. Each signal, or signal-sequence must be configured to a rack, a ____ in the rack, and the starting point or channel number? A. I/O B. SLOT C. BIT D. CARD. | Definition B. SLOT. |
| Term The macro commands that requre the USER key to be pressed alone (UK[1] - UK[7]) cannot have any instructions that _______ the robot? A. stop B. changes C. move D. limit. | Definition C. move. |
| Term Which I/O is used when using the EE connector on the robot? A. digital B. robot C. group D. analog. | Definition B. robot. |
| Term [image] A Macro that contains a motion instruction can be assigned to Teach Pendant User Key 1 without the shift key? TRUE OR FALSE. | Definition FALSE. |
| Term [image] What is the Binary Bit value for the following outputs? A. 17 B. 14 C. 6 D. 9. | Definition A. 17. |

Cards – Robotics Final – 73 cards

| Term (FANUC) Which of the below is the number one safety concern in a robotic system? A. people B. external devices C. robots D. tooling E. workpiece. | Definition A. people. |
| --- | --- |
| Term (FANUC) When an Emergency Stop button is pushed A. all digital I/O is turned off and the robot decelerates to a "soft stop" B. all digital I/O is turned off, then returned to their previous state when the program is resumed C. "dynamic" braking is employed on all joints and all brakes are engaged D. robot motion is stopped and the main power to the robot controller is disconnected.. | Definition C. "dynamic" braking is employed on all joints and all brakes are engaged. |
| Term (FANUC) The controller contains the computer that operates the robot. Which of the following is NOT housed within the controller? A. Memory B. Control Circuitry C. Operator Controls D. Servo Motors. | Definition D. Servo Motors. |
| Term (FANUC) Mode select key switch in the T1 position A. requires the robot to wait for a Programmable Logic Controller (PLC) command to start the currently selected program. B. is used when teaching robot positions in space. In this mode, speed is limited to 250 millimeters per second at 100%. C. is used when testing programs up to 2000 millimeters per second at 100% of maximum robot speed. D. is used when running full production. | Definition B. is used when teaching robot positions in space. In this mode, speed is limited to 250 millimeters per second at 100%.. |
| Term (FANUC) Which of these components shown in these pictures will allow high voltage to the robot? A. Circuit Breaker B. ON button C. OFF button D. Teach Pendant ON/OFF switch. | Definition A. Circuit Breaker. |
| Term (FANUC) The jog speed is selected by pressing and releasing the: A. FCTN key B. +%/-% keys C. arrow keys D. COORD keys. | Definition B. +%/-% keys. |
| Term (FANUC) Status indicator lights indicate the current status of the robot to an operator. These lights are: A. to the left of the LCD screen on the teach pendant -- iPendant B. at the base of the robot C. in the center of the teach pendants LCD screen display D. on the Standard Operators Panel (SOP). | Definition A. to the left of the LCD screen on the teach pendant -- iPendant. |
| Term (FANUC) If the HOLD status indicator is lit, which of the following does NOT occur? A. the program has been paused B. smooth deceleration has occurred C. power is maintained on the robot and execution can be resumed D. all control digital outputs are turned off. | Definition D. all control digital outputs are turned off. |
| Term Tool Center Point (TCP) can best be described as: A. the point at the center of the robot's faceplate on a newly purchased robot B. the location on the end of arm tooling where work will be performed. C. the point on the tool that is used to teach positions D. all of the above. | Definition D. all of the above. |
| Term (FANUC) Jogging the robot in the JOINT jog mode: A. moves the robot in a linear motion where precision is desirable B. moves the robot based on a Cartesian (XYZ) coordinate system C. allows the operator to jog each joint individually, or a combination of joints by pressing more than one jog key simultaneously D. prevents override speed from exceeding 15%. | Definition C. allows the operator to jog each joint individually, or a combination of joints by pressing more than one jog key simultaneously. |
| Term (FANUC) Based on the guidelines for naming teach pendant programs (TPP), which of the following names would be considered acceptable? A. PROG#1 B. 1PROG C. PROG1 D. PROG 1. | Definition C. PROG1. |
| Term (FANUC) Given the following teach pendant program motion instruction, which of the following robot motions will occur? L P[1] 1500mm/sec CNT75 A. joint motion at 50% override to position P[1] with fine motion termination B. a linear motion at 50% override to position P[1] with fine motion termination C. a linear motion at 1500 millimeters per second to position P[1] with continuous motion termination and 75% corner rounding D. a joint motion at 75% override to position P[1] with fine motion termination. | Definition C. a linear motion at 1500 millimeters per second to position P[1] with continuous motion termination and 75% corner rounding. |
| Term (FANUC) Every point in a robot work envelope is definable based on either degrees of rotation from center, or zero, for the JOINT jog mode, or distance from the robot's origin for Cartesian motion. Positional data is viewed by accessing the: A. status screen B. DATA key C. main menu D. position detail screen. | Definition D. position detail screen. |
| Term (FANUC) Positional data in the WORLD jog mode consists of location, orientation and configuration. What is the orientation of a position? A. the three values relative to the position of the x, y, and z axes and their distance from the zero position or origin measured in mm B. The three real values relative to the rotation of the tool center point about the x, y, and z axes, referred to as yaw, pitch, and roll expressed in degrees. C. the data structure consisting of values that reference the unique set of joint angles that are required to reach a point in space where there is more than one possible way to get there D. all of the above. | Definition B. The three real values relative to the rotation of the tool center point about the x, y, and z axes, referred to as yaw, pitch, and roll expressed in degrees.. |
| Term (FANUC) While on the JOINT position detail screen, the current position is definable based on: A. the number of joints moved simultaneously B. the degrees of rotation in either a positive or negative direction from zero C. default values appearing on the position detail screen D. the Cartesian coordinate system. | Definition B. the degrees of rotation in either a positive or negative direction from zero. |
| Term (FANUC) To modify the positional information of a taught position, jog the robot's tool center point (TCP) to the new position an press the: A. SAVE key in the FUNCTIONS menu B. SHIFT key and the corresponding function key for TOUCHUP C. INST key from the TPP teach pendant D. DEADMAN switch, enable the teach pendant, then press the RECORD pushbutton on the Standard Operator Panel. | Definition B. SHIFT key and the corresponding function key for TOUCHUP. |
| Term (FANUC) When needing to cursor along a line in a program to make changes to an element, pressing which option on the Function menu bar will display additional selection options for the element? A. LIST B. POSITION C. [CHOICE] D. EDIT. | Definition C. [CHOICE]. |
| Term (FANUC) To select an existing program to work with, operators should press the __________ key. A. MENUS B. NEXT C. SET UP D. SELECT. | Definition D. SELECT. |
| Term (FANUC) When step testing a program to verify the program's integrity: A. the STEP status indicator should be lit B. program statements may be tested independently in the forward (SHIFT+FWD) and backward (SHIFT+BWD) direction. C. the DEADMAN switch must be gripped and help and the Teach Pendant enabled D. all of the above. | Definition D. all of the above. |
| Term (FANUC) When continuously testing a program using the teach pendant: A. single step testing must be disabled B. it is not necessary to grip and hold the DEADMAN switch on the teach pendant C. a 5% default jog speed is automatically set within the system, therefore settings the jog speed is not necessary D. the program my be tested in a forward and backward direction using the SHIFT+FWD or SHIFT+BWD keystroke combination. | Definition A. single step testing must be disabled. |
| Term (FANUC) Placing the robot in LOCAL mode when the Teach Pendant is disabled: A. requires the robot to wait for a PLC command to start the currently selected program B. allows the operator to use the Cycle Start button on the SOP to start the currently selected program C. immediately begins robot motion for the currently selected program D. generates a Class III fault. | Definition B. allows the operator to use the Cycle Start button on the SOP to start the currently selected program. |
| Term (FANUC) Which teach pendant key do you press to abort a program? A. MENUS B. SELECT C. DATA D. FCTN. | Definition D. FCTN. |
| Term (FANUC) When jogging the robot, select the key on the Teach Pendant that must be pressed and held before you press a desired jog key. A. RESET B. SHIFT C. DEADMAN SWITCH D. COORD. | Definition B. SHIFT. |
| Term (FANUC) To Cycle start a program from the Standard Operator Panel the REMOTE/LOCAL keyswitch is placed into the: A. ON position. B. LOCAL position. C. REMOTE position. D. position it was in when step testing the program.. | Definition B. LOCAL position.. |
| Term (FANUC) The "right-hand rule" in a Cartesian Coordinate System: A. clearly defines a 2-dimensional space based on the x and y axis system. B. is helpful when determining the direction a robot will move while jogging in the WORLD. C. is a useful tool while jogging in the JOINT jog mode. D. is a selectable option which must be installed during a Cold Start. | Definition B. is helpful when determining the direction a robot will move while jogging in the WORLD.. |
| Term (FANUC) What is the Teach Tool? A. Allows you to "drag" the EOAT. B. Teaches the robot - similar to creating a program. C. Using the Teach Pendant, create a program. D. Using the Teach Pendant, jog the robot.. | Definition A. Allows you to "drag" the EOAT.. |
| Term (FANUC) To add a Part to the workcell: A. Select "Add a Part" from the Edit menu. B. Select "Add a Part" from the Tools menu. C. Select "Add a Part" from the Cell Browser within the Parts category. D. Select "Add a Part" from the File menu. | Definition C. Select "Add a Part" from the Cell Browser within the Parts category.. |
| Term (FANUC) To add End-of-Arm Tooling to the robot: A. Select "UT:#(Eoat)" within the Tooling category of the Cell Browser. B. From the Robot menu, select "Add End-of-Arm Tooling". C. From the Tools menu select "Add End-of-Arm Tooling". D. From the File menu select "Edit End-of-Arm Tooling". | Definition A. Select "UT:#(Eoat)" within the Tooling category of the Cell Browser.. |
| Term (FANUC) To adjust the view of the robot use the zoom in and out command which is: A. RIGHT mouse drag. B. LEFT mouse drag. C. BOTH mouse (left and right) drag or use the mouse wheel. D. Hold down the SHIFT+RIGHT drag.. | Definition C. BOTH mouse (left and right) drag or use the mouse wheel.. |
| Term (FANUC) To add a Pick Fixture to the Workcell: A. From the File menu select "Add a Fixture". B. From the Edit menu select "Add a Fixture". C. From the Cell menu, select "Add a Fixture" or Cell Browser, right click on Fixtures D. From the Tools menu, select "Add a Fixture". | Definition C. From the Cell menu, select "Add a Fixture" or Cell Browser, right click on Fixtures. |
| Term (FANUC) To remove a Fixture A. Click to select the Fixture within the Cell and from the keyboard press Delete. B. Within the Cell Browser, right click on the Fixture name, then select Delete Fixture. C. From the Edit menu, select remove Fixture D. All the above.. | Definition D. All the above.. |
| Term (FANUC) To create a simulated program: A. From the Teach menu select "Add Simulation Program". B. From the Process Navigator within the Teach TP Programs, select Add a TP Program. C. Within the Cell Browser, right click on Programs and select Add Simulation Program. D. All the above.. | Definition D. All the above.. |
| Term (FANUC) To run and view a program from beginning to end utilizing the EOAT pick and place a part: A. Right click on the program within the Cell Browser and select Run Program. B. From the Test-Run menu, select Run Panel C. Right click on the robot and select Run Program D. Click on Cycle Start within the icon bar.. | Definition D. Click on Cycle Start within the icon bar.. |
| Term (FANUC) To create an AVI file: A. Click on the Record button within the icon bar. B. From the Tools menu select "Create an AVI file" C. From the Robot menu select "Create an AVI file". D. From the File menu select "Create an AVI file".. | Definition A. Click on the Record button within the icon bar.. |
| Term (FANUC) For simulation to pickup and drop a part: A. You must define a close gripper in the EOAT. B. You must place a check in the "Allow part to be picked" and in the "Allow part to be placed" found within the Fixture Simulation tab. C. Adjust the part to the Tool Center Point. D. You must place a check in the "Allow part to be picked" and in the "Allow part to be placed" within the properties page of the part.. | Definition B. You must place a check in the "Allow part to be picked" and in the "Allow part to be placed" found within the Fixture Simulation tab.. |
| Term (FANUC) How many jog frames are standard on the R-J3 series controller? A. 5 B. 2 C. 3 D. 4. | Definition A. 5. |
| Term (FANUC) Select all methods that a Jog Frame can be taught. A. 3 point method B. 6 point method C. 2 point method D. direct entry E. 7 point method. | Definition A. 3 point method D. direct entry. |
| Term (FANUC) What are the 3 methods used to define a tool frame. A. 4, 3, and six point method B. 3, 6, and direct entry method C. 2, 4, and 6 point method D. 2, 3, and direct entry. | Definition B. 3, 6, and direct entry method. |
| Term (FANUC) What is the default location of the tool center point? A. at the center of the faceplate B. the center of the user frame C. the job frame D. the center of the robot. | Definition A. at the center of the faceplate. |
| Term (FANUC) What are the three methods used to define a RTCP frame? A. 2 and 6 point method B. 3 and 7 point method C. 3 point, 4 point and direct entry method D. all of the above. | Definition C. 3 point, 4 point and direct entry method. |
| Term (FANUC) How many user frames are standard on the R-J3 series controller? A. 13 B. 9 C. 8 D. 11. | Definition B. 9. |
| Term (FANUC) The status line describes the current status of the I/O. When the status displays PENDING, this means: A. an assignment has not been made B. the assignment is valid, but not active C. the assignment is valid and active D. the assignment is invalid based on the I/O hardware present when the controller was turned ON. | Definition B. the assignment is valid, but not active. |
| Term (FANUC) Which I/O is used when using the EE connector on the robot? A. digital B. robot C. analog D. group. | Definition B. robot. |
| Term (FANUC) When group output binary is set to 7, what DO[#]'s will be switched on? DO[1] = 1 DO[2] = 2 DO[3] = 4 D0[4] = 8 DO[5] = 16 A. 3, 4 B. 1, 2, 3 C. 2, 5 D. 2, 3, 4. | Definition B. 1, 2, 3. |
| Term (FANUC) An analog I/O signal is an input or output voltage that has a value within the range of the I/O board or module that is being used. A. true B. false. | Definition A. true. |
| Term (FANUC) When inserting a program instruction, you must highlight either a line number or END. A. true B. false. | Definition A. true. |
| Term (FANUC) Timers must start and end in the same program. A. true B. false. | Definition B. false. |
| Term (FANUC) When a CALL statement branches from the main program that executes the entire program, you must insert another CALL statement to return back to the main program. A. true B. false. | Definition B. false. |
| Term (FANUC) Position register element instruction allows you to change an individual value in a joint or Cartesian position. A. true B. false. | Definition A. true. |
| Term (FANUC) The following display is a direct or indirect addressing: R[3]=2 A. direct B. indirect. | Definition A. direct. |
| Term (FANUC) The renumber function reassigns all position numbers in the program so that they are in sequential order. Renumbering changes the position ID, not the positional information. A. true B. false. | Definition A. true. |
| Term (FANUC) will add lines exactly as they were copied and retains the current position numbers. | Definition F3 - POS ID. |
| Term (FANUC) will add the lines exactly as they were copied, but leaves the position numbers blank. | Definition F2 - LOGIC. |
| Term (FANUC) will add lines exactly as they were copied and renumbers the copied positions with the next available position number. | Definition F4 - POSITION. |
| Term (FANUC) speed and termination types are pasted in the proper instructions to maintain the original speed and termination of each path segment. | Definition F3 -- RM-POS-ID. |
| Term (FANUC) will add lines in reverse ore and does not record the positions, but leaves the position numbers blank. | Definition F1 -- R-LOGIC. |
| Term (FANUC) this is exactly like the RM-POD-ID except that new position numbers are assigned in the pasted lines. | Definition F5 -- RM-POS. |
| Term (FANUC) will add lines in reverse order and retains their original position numbers. | Definition F2 -- R-POS-ID. |
| Term (FANUC) will add lines in reverse order and renumbers the copied positions with the next available position numbers. | Definition F4 -- R-POS. |
| Term (FANUC) The replace option will help you to modify registers, motion speed and termination, inputs and outputs, jump labels, calls and time before and time after. A. true B. false. | Definition A. true. |
| Term (FANUC) A macro that contains a motion instruction can be assigned to teach pendant user key 1 without the shift key. A. true B. false. | Definition B. false. |
| Term (FANUC) A macro that contains a motion instruction can be assigned to user 1 button on the standard operator's panel when using R-J3iB and earlier controllers. A. true B. false. | Definition A. true. |
| Term (FANUC) When assigning a number for the standard operator user key 1 for controllers R-J3iB and earlier, you can use SP[4]. A. true B. false. | Definition A. true. |
| Term (FANUC) When assigning user key SU[3}, the key that executes the macro is: A. shift+tool1 B. shift+move menu C. shift+I/O D. shift+tool2. | Definition B. shift+move menu. |
| Term (FANUC) What start up method is used when turning on power to the robot and controller without entering boot monitor functions? A. hot start B. cold start. | Definition A. hot start. |
| Term (FANUC) Can you always start the same program from the SOP cycle start while in remote mode? A. yes B. no. | Definition B. no. |
| Term (FANUC) Each production check provides different enabled choices within the program select detail. Which condition takes priority when multiple conditions are enabled? A. "check when run" B. "check when resume" C. "force condition". | Definition C. "force condition". |
| Term (FANUC) When the controller is in local mode it cannot start from an external signal such as a PLC or cell controller. A. true B. false. | Definition A. true. |
| Term (FANUC) When you enable the AT HOME check within the setup menu, the controller will check to see if the robot is at the home position and if it's not at home position then it will not run the production program. A. true B. false. | Definition A. true. |
| Term (FANUC) You must turn the Teach Pendant ON in order to copy an existing program. A. true B. false. | Definition B. false. |
| Term (FANUC) If you are saving files to a disk, you must set the default device. Which of the following menu items would you find this? A. SETUP/DEVICE B. FILE/UTILITIES C. SELECT/UTILITIES D. USER/SETTINGS. | Definition B. FILE/UTILITIES. |
| Term (FANUC) Teach Pendant programs can be loaded from the FILE menu and one other menu. What is the other screen? A. SETUP menu B. SELECT menu C. UTILITIES menu D. USER menu. | Definition B. SELECT menu. |
| Term (FANUC) To display the contents of a Device, you must Generate a Directory. A. true B. false. | Definition A. true. |

## Cards – Fanuc Pretest Handling Pro – 25 cards

| Term Turning the teach pendent on: A) Activates the default program B) Enables Communication with External Devices C) Gives motion control soley to the operator via the TP D) Energizes robotic arm. | Definition C) Gives motion control soley to the operator via the TP. |
| --- | --- |
| Term What must be accomplished to avoid hard-stops and over travels? A) Set UtOOL offsets B) Set axis Limits C) Set UFRAM offsets D Move each Robot joint +/- 20 degrees and reset. | Definition B) Set axis Limits. |
| Term SHIFT and the hard key COORD will display the following menu: A) The position menu B) The utilities menu C) The jog menu D) The manual functions menu. | Definition C) The jog menu. |
| Term SHIFT and POINT are needed to do what? A) Change Robot Speed B) Request the positional screen to appear C) Record the current position of the robot D) Execute a MACRO program and return the robot to home. | Definition C) Record the current position of the robot. |
| Term In the motion command "J P[2]100% FINE", if you wanted to change the "FINE" component to a "CNT100" what buttons would you press? A) FCTN followed by ABORT ALL B) SHIFT and FWD C) SELECT followed by NEXT D) Select component, then press [CHOICE]. | Definition D) Select component, then press [CHOICE]. |
| Term To switch manually from one program to another, you have to be in which screen? A) Select B) Edit C) Status D) Position. | Definition A) Select. |
| Term How many Robot I/O point are standered on a controller?. | Definition 16. |
| Term Which soft key is pressed to display the current motion instruction defaults from the EDIT screen? A) POINT B) NEXT C) POSN D) [INST]. | Definition A) POINT. |
| Term When creating a program, which hard key should be pressed? A) FCTN B) FWD C) Select D) Hold. | Definition C) Select. |
| Term When using the Prog Adjust utility, which statement is correct? A) All adjustments are relative to the TOOL Frame B) Pressing CLR_ADJ or CLR_ALL will save the curent enabled data values C) The maximum single linear adjustment allowed is 100mm D) Program Adjust is performed from the Status Setup Screen. | Definition B) Pressing CLR_ADJ or CLR_ALL will save the curent enabled data values. |
| Term Which of these tasks can be performed from the File menu? A) Excute a program from a storage device B) Get a directory of the default device C) List the program names for execution, teachng data, or editing D) Print a program found on a disk. | Definition B) Get a directory of the default device. |
| Term If while jogging the robot, you get an "In singularity" error, what must you do? A) Jog the robot in Tool B) Change the correct user frame C) Joint jog J5 +/- at least 10 degrees D) Press the position key and reset the axis limits. | Definition C) Joint jog J5 +/- at least 10 degrees. |
| Term The "Program Header Information" at the DETAIL softkey enables you to modify the following program information, except: A) Program Name B) Group Mask C) Activate Tool Frame D) Program Sub Type. | Definition C) Activate Tool Frame. |
| Term What is the suggested type of motion used to move the robot to a Home position? A) Linear B) Simultaneous C) Joint (faster) D) Circular. | Definition C) Joint (faster). |
| Term Which softkey allows you to enter additional program instructions into a TP program? A) [ED_DEF] B) EDIT C) DETAIL D) [INST]. | Definition D) [INST]. |
| Term Which of the following is a type of I/O signal? A) UIP I/O B) SIP I/O C) Digital I/O D) Configuration I/O. | Definition C) Digital I/O. |
| Term Which of the following hard keys would bring the menu item ABORT ALL to TP screen? A) FCTN B) HOLD C) RESET D) E-STOP. | Definition A) FCTN. |
| Term If you would like to look at the positional data of P[3] in the instruction "J P[3] 100%FINE", what buttons would you press? A) Press the DATA hardkey then F1[TYPE] and choose 2 for PR[] B) Pres POSN hardkey to see the data directly C) Cursor to the 3 pf P[3] and press F5 POSITION D) Press STATUS, NEXT, and F2 DETAIL. | Definition C) Cursor to the 3 pf P[3] and press F5 POSITION. |
| Term What are the two menus from which you can save TP files? A) SELECT & FILE B) SELECT & DATA C) SYSTEM & FILE D) ED_CMD & CONFIG. | Definition A) SELECT & FILE. |
| Term Which is not a valid way to execute an assigned MACRO? A) Pressing TP user key B) Pressing the "USER 1" button on the SOP C) Executing an item from the Manual functions screen D) Holding PREV & NEXT. | Definition D) Holding PREV & NEXT. |
| Term Which instruction will increment Register 7 (R[7]) by 1? A) R[7]=1 B) R[7]+1 C) R[7]++ D) R[7]=R[7]+1. | Definition D) R[7]=R[7]+1. |
| Term What instruction will send program flow out to a sub-program? A) A) JMP/LBL B) REMARK C) CALL D) SELECT. | Definition C) CALL. |
| Term What are the two (2) possible actions of an IF?SELECT statement? A) SELECT & Set an OUPUT B) JMP/LBL & SELECT C) Set an OUTPUT & CALL D) CALL & JMP/LBL. | Definition D) CALL & JMP/LBL. |
| Term To swich between JOINT, WORLD, TOOL, JGFRM, & USER for jogging purposes, you need to press which of the following keys?. | Definition COORD. |
| Term Where is Position Register (PR[]) data valid? A) Current program only B) Globally C) Nowhere D) Max 2 programs. | Definition B) Globally. |

## Cards – Robotics Final 194

| Term (Chap 1) Machines that can be reprogrammed to meet changing manufacturing needs are an example of __________. A. flexible automation B. hard automation C. artificial intelligence D. anthropomorphic robots. | Definition A. flexible automation. |
| --- | --- |
| Term (Chap 1) An intelligent robot uses __________ to detect changes in the work environment. A. human operators B. numerical data C. sensors D. None of the above. | Definition C. sensors. |
| Term (Chap 1) The RIA definition of an industrial robot states that it is a machine, is reprogrammable, is flexible, and _______. A. has a multifunction manipulator B. is intelligent C. is more efficient than human operators D. All of the Above. | Definition A. has a multifunction manipulator. |
| Term (Chap 1) The LRMate CERT cart has a sensor on the work cell door. When is it active? A. any time power is applied to the CERT cart B. only when using the teach pendant C. only when the robot is in AUTO mode D. only when interfaced with another robot. | Definition C. only when the robot is in AUTO mode. |
| Term (Chap 1) A(n) __________ is the term originally used for what we now consider to be a robot.. | Definition Automaton. |
| Term (Chap 1) A robot that can be given new instructions to perform new tasks is considered __________.. | Definition reprogrammable. |
| Term (Chap 1) A(n) __________ automation machine is specifically designed for particular tasks on an assembly line.. | Definition hard. |
| Term (Chap 1) Science fiction writer Isaac Asimov coined the term __________, which means the use of robots.. | Definition robotics. |
| Term (Chap 1) A(n) __________ robot has a humanlike form.. | Definition anthropomorphic. |
| Term (Chap 1) On the LRMate CERT training cart, DCS stands for: __________ __________ __________.. | Definition Dual Control Safety. |
| Term (Chap 1) EOAT stands for _____ _____ _____ _____.. | Definition End of Arm Tooling. |
| Term (Chap 1) What does the acronym SCARA stand for?. | Definition Selective Compliant Assembly Robot Arm. |
| Term (Chap 1) Explain the benefits and drawbacks of hard automation machinery.. | Definition Good for high-speed manufacturing with repetitive motions. One-task oriented..... meaning useless once product changes.. |
| Term (Chap 1) List three common safety enhancing devices used for a robotic work cell.. | Definition Pressure mats Light curtains E-stops Hard Stops. |
| Term (Chap 1) What is the number one safety rule for entering a robotic work cell?. | Definition Have the Teach Pendant in your control.. |
| Term (Chap 1) Describe collision guard for the LRMate CERT cart.. | Definition |
| Term (Chap 1) Describe four steps for turning on the LRMate CERT cart.. | Definition |
| Term (Chap 1) Describe robot payload.. | Definition End Effector + Workpiece. |
| Term (Chap 1) What is a payload schedule and what is it used for?. | Definition Total weight of the product being carried and the robot's own components. Used to tell the robot how to manipulate the product.. |
| Term (Chap 2) The five main components of an industrial robot include which of the following? A. Pneumatic drive. B. Tachometer C. Work envelope. D. Means for programming.. | Definition D. Means for programming.. |
| Term (Chap 2) The highest hierarchical control level is __________ control. A. actuator B. main C. path D. base. | Definition A. actuator. |
| Term (Chap 2) The __________ provides the energy to drive the robot's controller and actuators. A. program B. power supply C. servo amplifier D. None of the above.. | Definition B. power supply. |
| Term (Chap 2) Which of the following is an example of a means for programming? A. Teach pendant. B. Teach box. C. Hand-held programmer. D. All of the above.. | Definition D. All of the above.. |
| Term (Chap 2) The vertical traverse provides __________ motion of a robot's arm. A. up-and-down B. side-to-side swivel C. extending and retracting D. point-to-point. | Definition A. up-and-down. |
| Term (Chap 2) In a(n) __________ system, no feedback mechanism is used to compare programmed position to actual positions. A. servo B. open-loop C. closed-loop D. direct-drive. | Definition B. open-loop. |
| Term (Chap 2) A robot arm using the __________ configuration can start and stop simultaneously along the X, Y, and Z axes. A. cylindrical B. Cartesian C. revolute D. spherical. | Definition B. Cartesian. |
| Term (Chap 2) The __________ configuration consists of two orthogonal slides places at 90 degree angle and mounted on a rotary axis. A. cylindrical B. Cartesian C. revolute D. spherical. | Definition A. cylindrical. |
| Term (Chap 2) The five major components of industrial robots are the controller, manipulator, end effector, base, and workpiece. A. true B. false. | Definition B. false. |
| Term (Chap 2) Robot movement is initiated by a series of instructions stored in the end effector. A. true B. false. | Definition B. false. |
| Term (Chap 2) Hierarchical control assigns levels of organization to the controllers within a robotic system. A. true B. false. | Definition A. true. |
| Term (Chap 2) A non-servo robot is a closed-loop system. A. true B. false. | Definition B. false. |
| Term (Chap 2) A servo amplifier translates signals from the controller into motor voltage and current signals. A. true B. false. | Definition A. true. |
| Term (Chap 2) The three basic levels of hierarchical control are actuator control, path control, and __________ control.. | Definition main. |
| Term (Chap 2) The arm of a robot that moves materials, parts, or tools is the __________.. | Definition manipulator. |
| Term (Chap 2) The __________ is the robot's hand.. | Definition end effector. |
| Term (Chap 2) The extending and retracting movement of a robot arm is an example of __________ traverse motion.. | Definition radial. |
| Term (Chap 2) The simplest type of robot is a(n) __________ robot, also called a limited sequence robot.. | Definition non-servo. |
| Term (Chap 2) The __________ configuration offers flexible reach, a large work envelope, and anthropomorphic movement.. | Definition revolute. |
| Term (Chap 2) The movement of a(n) __________ configuration robot resembles the action of the turret on a military tank.. | Definition spherical. |
| Term (Chap 2) Describe the types of testing available on the LRMate CERT robot and identify when you'd use them.. | Definition - Single Step - Auto Low - Auto Full Speed. |
| Term (Chap 2) What does TCP stand for?. | Definition Tool Center Point. |
| Term (Chap 2) Before inspecting a robot you should always release stored energy. This energy can be in the form of:. | Definition Pneumatic, hydraulic, and/or electric energy.. |
| Term (Chap 2) List and describe the three positions of the mode select switch on the R-30iA controller.. | Definition Auto - run programs T1 - limit movement to 250 mm/sec T2 - 100% motion possible. |
| Term (Chap 2) List and describe three common pre-defined positions used in a robotic work cell.. | Definition Home - A position out of the work path. Service - Best position for robot servicing. Start - Position ready to begin sequence.. |
| Term (Chap 3) Using a great number of points in __________ motion creates movement that is smooth and continuous. A. pick-and-place B. point-to-point C. continuous-path D. None of the above.. | Definition C. continuous-path. |
| Term (Chap 3) The first robot programming language was known as __________. A. AML B. WAVE C. BASIC D. Pascal. | Definition B. WAVE. |
| Term (Chap 3) In __________ programming, each level accepts commands from the level above and generates simplified commands for the level below. A. hierarchical control B. off-line C. walk-through D. task-level. | Definition A. hierarchical control. |
| Term (Chap 3) A(n) __________ system uses recognizable words as a form of audio data entry. A. voice recognition B. speech recognition C. artificial intelligence D. Both A and B.. | Definition D. Both A and B.. |
| Term (Chap 3) A device that prevents an end effector from moving beyond a certain point is a(n) __________. A. compiler B. end stop C. manual rate control box D. subroutine. | Definition B. end stop. |
| Term (Chap 3) The __________ generation of robots evolved through the use of artificial intelligence. A. first B. second C. third D. fourth. | Definition C. third. |
| Term (Chap 3) Limited-sequence robots use pick-and-place motion to move the end effector to the correct position. A. true B. false. | Definition A. true. |
| Term (Chap 3) In continuous-path motion, an operator leads the robot through the arias positions involved in an operation. A. true B. false. | Definition B. false. |
| Term (Chap 3) In teach pendant programming, an operator adjusts end stops, switches, and hoses to set up the motion sequence. A. true B. false. | Definition B. false. |
| Term (Chap 3) Walk-through programming is used for continuous-path robots. A. true B. false. | Definition A. true. |
| Term (Chap 3) Off-line programming is performed away from the robot, using a computer. A. true B. false. | Definition A. true. |
| Term (Chap 3) Because more programmed points are used, pick-and-place motion is smoother than continuous-path motion. A. true B. false. | Definition B. false. |
| Term (Chap 3) Hierarchical control systems use sensory feedback to close control loops. A. true B. false. | Definition A. true. |
| Term (Chap 3) The __________ generation of industrial robots performed purely repetitive tasks and did not respond to changing condition.. | Definition first (1st). |
| Term (Chap 3) _________ is the science and engineering of making machines perform operations commonly associated with intelligent human behavior.. | Definition Artificial Intelligence (A.I.). |
| Term (Chap 3) A(n) __________ is a set of instructions within a program.. | Definition subroutine. |
| Term (Chap 3) The path of motion for _________ robots is a series of straight lines between hundreds of possible programmed points, with no real control of the end effector's movement between the points.. | Definition Point-to-Point (P-T-P). |
| Term (Chap 3) With __________ motion, control of the end effector's path is more important than the end point positioning.. | Definition continuous path. |
| Term (Chap 3) __________ programming systems offer interactive screens that create, teach, debug, and run application programs.. | Definition Task level. |
| Term (Chap 3) Explain how point-to-point robots are programmed.. | Definition |
| Term (Chap 4) Designing parts of a product for eases of robotic assembly is __________. A. production planning B. repeatability C. dynamic performance D. designing for manufacturability. | Definition D. designing for manufacturability. |
| Term (Chap 4) The relationship of the __________ to work fixtures must be taken into account when designing a work station. A. end effector B. operator C. power supply D. None of the above.. | Definition A. end effector. |
| Term (Chap 4) A(n) __________ safely contains dislodged workpieces. A. light curtain B. interlock C. safety fence D. pressure sensitive safety mat. | Definition C. safety fence. |
| Term (Chap 4) Greater flexibility may be achieved by mounting a robot on the __________ within a work envelope, instead of the floor. A. wall B. ceiling C. conveyor system D. Both A and B.. | Definition D. Both A and B.. |
| Term (Chap 4) The number of __________ determines a robot's ability to move within the work envelope. A. end effectors B. degrees of freedom C. axes D. controllers. | Definition B. degrees of freedom. |
| Term (Chap 4) The accuracy of movement of a robot's tool tip is described in terms of __________. A. command resolution B. repeatability C. spatial resolution D. dynamic performance. | Definition C. spatial resolution. |
| Term (Chap 4) The maximum weight of material a robot is capable of handling on a continuous basis is the __________. A. dynamic performance B. payload C. command resolution D. Both A and C. | Definition B. payload. |
| Term (Chap 4) Providing a safe working environment is considered only after a robotic system is designed and installed. A. true B. false. | Definition B. false. |
| Term (Chap 4) In toxic applications, interlocks protect personnel from chemicals. A. true B. false. | Definition B. false. |
| Term (Chap 4) Robots with six degrees of freedom can work anywhere within their work envelope. A. true B. false. | Definition A. true. |
| Term (Chap 4) Injection molding is an example of a pick-and-place operation. A. true B. false. | Definition B. false. |
| Term (Chap 4) An infrared light curtain is programmable to encompass certain areas and exclude others. A. true B. false. | Definition A. true. |
| Term (Chap 4) Force-sensing equipment can identify contact with a workpiece and adjust tools for constant pressure. A. true B. false. | Definition A. true. |
| Term (Chap 4) Two primary safety devices used with industrial robots are __________ and sensing systems.. | Definition barriers. |
| Term (Chap 4) A(n) __________ automatically turns off dangerous machinery when a worker steps on the carpet.. | Definition pressure mat. |
| Term (Chap 4) The weight of the end effector is commonly included in the __________ rating.. | Definition payload. |
| Term (Chap 4) __________ operations involve pumping molten metals into closed dies.. | Definition Die casting. |
| Term (Chap 4) Continuous-path programming must be used for __________ operations to ensure smooth motion.. | Definition spraying. |
| Term (Chap 5) A(n) __________ system transfers power from one point to another through mechanical motion that is used to do work. A. sensing B. optoelectronic C. timing D. electromechanical. | Definition D. electromechanical. |
| Term (Chap 5) __________ devices operate within the transmission path and later the flow of power. A. Control B. Load C. Cycle D. Commutator. | Definition A. Control. |
| Term (Chap 5) The __________ unit of an industrial robot determines its flexibility and efficiency. A. timing B. control C. sensing D. power. | Definition B. control. |
| Term (Chap 5) In a closed-loop system, the interaction between the control unit and the load is called __________. A. feedback B. cycle timing C. comparator D. slip. | Definition A. feedback. |
| Term (Chap 5) The __________ is the rotating component of a motor and includes the armature, shaft, and associated parts. A. stator B. rotor C. commutator D. detector. | Definition B. rotor. |
| Term (Chap 5) A __________ dc motor has two sets of field windings. A. shunt-wound B. permanent-magnet C. series-wound D. compound-wound. | Definition D. compound-wound. |
| Term (Chap 5) Machines that change the position or speed of a mechanical object are __________ systems. A. servo B. synthesized C. synchronous D. timing. | Definition A. servo. |
| Term (Chap 5) DC stepping motors are primarily used to change electrical pulses into ___________. A. dc power B. feedback signals C. rotary motion D. input commands. | Definition C. rotary motion. |
| Term (Chap 5) The most common source of power for synthesized systems is alternating current (ac). A. true B. false. | Definition A. true. |
| Term (Chap 5) A series-wound motor is a type of ac motor. A. true B. false. | Definition B. false. |
| Term (Chap 5) The counter electromotive force flows against the voltage coming into the motor. A. true B. false. | Definition A. true. |
| Term (Chap 5) The transmission path provides a channel for the transfer of energy. A. true B. false. | Definition A. true. |
| Term (Chap 5) In bifilar construction, three separate wires are simultaneously wound into the stator coil slots. A. true B. false. | Definition B. false. |
| Term (Chap 5) Flux is the change in direction of electron flow in alternating current power. A. true B. false. | Definition B. false. |
| Term (Chap 5) In a mechanical system, __________ measure physical quantities, such as pressure, speed, and force.. | Definition sensors. |
| Term (Chap 5) A(n) __________ motor can be powered by either an ac or dc source and is commonly used for portable tools and small equipment.. | Definition universal. |
| Term (Chap 5) In an automated system, the part (or parts) designed to produce work is referred to as the __________.. | Definition load. |
| Term (Chap 5) As the armature of a dc motor rotates, it generates its own voltage called __________.. | Definition cemf (counter electro-motive force). |
| Term (Chap 5) A(n) __________ rotor is the solid rotor in a single-phase induction motor.. | Definition squirrel cage. |
| Term (Chap 5) How is the end effector protected against damage from overload?. | Definition Mechanical fuses or overload springs.. |
| Term (Chap 6) In __________ systems, air or liquid, or a combination of both, is used to transfer power. A. fluid power B. hydraulic C. pneumatic D. All of the above.. | Definition D. All of the above.. |
| Term (Chap 6) In a fluid power system, the __________ is the component that is ultimately controlled. A. pump B. load C. fluid motor D. force. | Definition B. load. |
| Term (Chap 6) In a pneumatic system, the __________ holds pressurized air and serves as the reservoir. A. FRL unit B. transmission line C. reciprocating pump D. storage tank. | Definition D. storage tank. |
| Term (Chap 6) The process of removing contaminants from pressurized air is called __________. A. filtering B. conditioning C. metering D. pressurizing. | Definition B. conditioning. |
| Term (Chap 6) In fluid power system, friction is called __________. A. resistance B. heat exchange C. force D. inertia. | Definition A. resistance. |
| Term (Chap 6) The amount of force applied to a specific area is __________. A. turbulence B. flow C. pressure D. resistance. | Definition C. pressure. |
| Term (Chap 6) Which of the following is not an example of a heat exchange device? A. force-air fan unit. B. water-jacket cooler. C. gaseous cooling unit. D. FRL unit. | Definition D. FRL unit. |
| Term (Chap 6) __________ control devices are used to start, stop, or reverse fluid flow without causing a significant change in pressure or flow rate. A. Pressure B. Direction C. Flow D. Temperature. | Definition B. Direction. |
| Term (Chap 6) Controlling the rate of fluid flow is known as __________. A. metering B. conditioning C. filtering D. pressurizing. | Definition A. metering. |
| Term (Chap 6) A(n) __________ system produces mechanical energy by combining fluid power and electrical power systems. A. hybrid B. pneumatic C. hydraulic D. efficient. | Definition A. hybrid. |
| Term (Chap 6) Direction control devices alter the volume or flow rate of the fluid. A. true B. false. | Definition B. false. |
| Term (Chap 6) Pascal's law states that pressure applied to a convinced fluid is transmitted, undiminished, throughout the fluid. A. true B. false. | Definition A. true. |
| Term (Chap 6) A check valve permits fluid flow in only one direction. A. true B. false. | Definition A. true. |
| Term (Chap 6) Pressure is the gravitational force exerted on a body by the Earth. A. true B. false. | Definition B. false. |
| Term (Chap 6) A hydraulic pump operates intermittently and forces compressed air into the storage tank. A. true B. false. | Definition B. false. |
| Term (Chap 6) Multiplication of force allows a force as small as a few ounces to control an object of several tons. A. true B. false. | Definition A. true. |
| Term (Chap 6) All fluid power system consist of an energy source, transmissions path, load, controls, and __________.. | Definition indicators. |
| Term (Chap 6) __________ is any factor that tends to produce or modify the motion of an object.. | Definition Force. |
| Term (Chap 6) __________ actuators produce a limited amount of twisting or turning motion in either direction.. | Definition Rotary. |
| Term (Chap 6) What are the processes involved in conditioning fluid for a pneumatic system?. | Definition |
| Term (Chap 7) A sensor operates by converting light, heat, or mechanical energy into __________. A. movement B. electrical energy C. work D. a magnetic field. | Definition B. electrical energy. |
| Term (Chap 7) Wavelengths of light are measured in __________. A. degrees B. nanometers C. volts D. Megahertz. | Definition B. nanometers. |
| Term (Chap 7) An opto-electronics device that converts light energy into electrical energy is a __________. A. photovoltaic device B. solar cell C. photoconductive device D. Both A and B. | Definition D. Both A and B. |
| Term (Chap 7) Computer vision sensors use __________ to determine the distance to an object based on the apparent size of the camera image. A. triangulation B. visual servoing C. stadimetry D. capacitance. | Definition C. stadimetry. |
| Term (Chap 7) An eddy current proximity sensor produces __________ in the small space of a detector unit. A. sound waves B. magnetic field C. infrared light D. dc current. | Definition B. magnetic field. |
| Term (Chap 7) Sensors restrict robot intelligence by slowing down processes. A. true B. false. | Definition B. false. |
| Term (Chap 7) All objects emit infrared thermal radiation. A. true B. false. | Definition A. true. |
| Term (Chap 7) Sound sensors detect spatial relationships and provide depth information. A. true B. false. | Definition B. false. |
| Term (Chap 7) Magnetic field sensors identify a change in a magnetic field without making physical contact with objects in the environment. A. true B. false. | Definition A. true. |
| Term (Chap 7) LEDs are solid-state opto-electronics devices. A. true B. false. | Definition B. false. |
| Term (Chap 7) An LVDT is a type of capacitive transducer. A. true B. false. | Definition B. false. |
| Term (Chap 7) __________ sensors are used for applications such as color measurement, leak detection, and impurity detection.. | Definition Ultraviolet. |
| Term (Chap 7) In a ruby laser beam, __________ travel along the axis of the ruby rod and are reflected by mirrors on each end, which amplifies the light.. | Definition photons. |
| Term (Chap 7) The __________ output generated by a dc tachometer is translated into speed readings.. | Definition voltage. |
| Term (Chap 7) A(n) __________ is a type of tactile sensor with an actuator that is mechanically linked to a set of contacts, which are either normally open or normally closed.. | Definition limit switch. |
| Term (Chap 7) What is the piezoelectric effect?. | Definition Voltage added to a crystal causes it to vibrate.. |
| Term (Chap 8) Using the gibers to hold round objects is a __________ grip. A. palmar B. cylindrical C. spherical D. spread. | Definition C. spherical. |
| Term (Chap 8) A __________ gripper delivers 360 degrees of clamping contact. A. three-finger B. collet C. mechanical finger D. vacuum. | Definition B. collet. |
| Term (Chap 8) Collision sensors are attached to the __________ to protect equipment from being damaged during operation. A. robot arm B. gripper C. automatic tool changer D. axes. | Definition A. robot arm. |
| Term (Chap 8) Which of the below is NOT a prehensile movement? A. cylindrical grip B. spread C. palmar grip D. lateral grip. | Definition B. spread. |
| Term (Chap 8) Tools are end effectors that perform prehensile movements by grasping objects and moving them. A. true B. false. | Definition B. false. |
| Term (Chap 8) Overload sensors detect obstructions or overload conditions within fractions of a second. A. true B. false. | Definition A. true. |
| Term (Chap 8) A remote center compliance device is a type of collision sensor. A. true B. false. | Definition B. false. |
| Term (Chap 8) End effectors are devices attached to the wrist of a(n) __________.. | Definition multifunction manipulator. |
| Term (Chap 8) End effectors that perform prehensile movements to grasp and move objects are called __________.. | Definition grippers. |
| Term (Chap 8) A(n) __________ device should be installed to prevent damage to a robot's arm or wrist if the end effector becomes stuck during an operation.. | Definition breakaway. |
| Term (Chap 8) A(n) __________ gripper delivers 360 degrees of clamping contact.. | Definition collet. |
| Term (Chap 8) __________ is the end effector's ability to tolerate the misalignment of mating parts.. | Definition Compliance. |
| Term (Chap 8) A(n) __________ is a device that is installed in the wrist of the robot to help compensate for workpiece misalignment or irregularities.. | Definition RCC. |
| Term (Chap 8) A(n) __________ gripper uses a magnetic field to pick up an object.. | Definition electromagnetic. |
| Term (Chap 8) What factors must be addressed when designing a robotic system with changeable end effectors?. | Definition - Tool Center Point - Compliance - Programming. |
| Term (Chap 10) Digital output ports permit digital signals to be sent from a robot __________ to equipment controls. A. controller B. end effector C. power supply D. work envelope. | Definition A. controller. |
| Term (Chap 10) Machine vision systems use video cameras and computers to translate __________ into images. A. light energy B. programmed data C. operator input D. None of the above.. | Definition A. light energy. |
| Term (Chap 10) During __________, an analog-to-digital converter changes the analog signal of an image into an equivalent digital signal. A. image acquisition B. image preprocessing C. image analysis D. image interpretation. | Definition B. image preprocessing. |
| Term (Chap 10) Common I/O ports include interface and image analysis. A. true B. false. | Definition B. false. |
| Term (Chap 10) Limit switches communicate information through output ports. A. true B. false. | Definition B. false. |
| Term (Chap 10) Digital output ports allow a robot to interact with external equipment, such as a conveyor belt drive motor, solenoid, or another robot. A. true B. false. | Definition A. true. |
| Term (Chap 10) The common point at which two or more systems communicate is a(n) __________.. | Definition interface. |
| Term (Chap 10) Two computers can communicate with each other over telephone circuits via __________ ports connected to modems.. | Definition serial. |
| Term (Chap 10) Using algorithms, computer software identifies and measures features of a digital image during the image __________ process.. | Definition analysis. |
| Term (Chap 10) List the four functions that take place during image processing.. | Definition - Image Acquisition - Image PreProcessing - Image Analysis - Image interpretation. |
| Term (Chap 11) Most system problems are __________ failures. A. component B. power C. operator D. safety. | Definition A. component. |
| Term (Chap 11) A __________ solder should be used for all electronic circuit work. A. cold B. acid-core C. resin-core D. None of the above. | Definition C. resin-core. |
| Term (Chap 11) ___________ is key to preventative maintenance. A. Efficiency B. Testing equipment C. Inventory cycle D. Documentation. | Definition D. Documentation. |
| Term (Chap 11) Which of the following devices can be assigned macro instructions? A. MF[1] to MF[99] : Items on the manual operation screen B. UK[1] to UK[7] : User keys 1 to 7 on the teach pendant C. SU[1] to SU[7] : User keys 1 to 7 + SHIFT key on the TP D. All of the above. | Definition D. All of the above. |
| Term (Chap 11) Thorough circuit and system testing should be performed before a visual inspection. A. true B. false. | Definition B. false. |
| Term (Chap 11) To gather information about a particular system, a technician should reference a schematic, service or maintenance manual, operating manual, and an experienced operator. A. true B. false. | Definition A. true. |
| Term (Chap 11) Scrap is a manufactured part that is reconditioned and put back into service. A. true B. false. | Definition B. false. |
| Term (Chap 11) You can set a macro to run from a Teach Pendant user key alone or with the SHIFT key. A. true B. false. | Definition A. true. |
| Term (Chap 11) A macro instruction is a function for registering a program consisting of a sequence of instructions as one instruction, and calling such a set of instruction for execution as required. A. true B. false. | Definition A. true. |
| Term (Chap 11) When a key on the teach pendant is assigned to a macro instruction, it becomes that macro instructions device, and the key can still be used for its original function. A. true B. false. | Definition B. false. |
| Term (Chap 11) A system malfunction should be traced to a specific __________ within the equipment.. | Definition subsystem. |
| Term (Chap 11) An equipment part that cannot be fixed is __________.. | Definition scrap. |
| Term (Chap 11) Preventative maintenance programs place a great deal of responsibility on the __________.. | Definition operator. |
| Term (Chap 11) You can set up a macro command program from the Setup Macro Command Screen to be executed from the __________ Functions menu.. | Definition Manual. |
| Term (Chap 11) The item to which a macro instruction is assigned is called a __________.. | Definition device. |
| Term (Chap 11) What pieces of testing equipment are used to isolate a system problem?. | Definition - Multimeter - Oscilloscope - Circuit Diagram - Manual. |
| Term (Chap 12) Robots can improve product quality because of their.... A. Accuracy B. repeatability C. cost effectiveness D. Both A and B. | Definition D. Both A and B. |
| Term (Chap 12) The money a business spends to purchase fixed assets is a A. avoidance cost B. allocation C. capital investment D. market venture. | Definition C. capital investment. |
| Term (Chap 12) The money spent on replacing existing production or assembly methods to reduce current costs is an investment known as A. cost savings B. non-economic justification C. avoidance cost D. re-tooling. | Definition A. cost savings. |
| Term (Chap 12) Intelligent machines can do the same job as trained workers and save labor costs by replacing those worker. A. true B. false. | Definition B. false. |
| Term (Chap 12) Automated equipment can easily compensate for irregularities in product components. A. true B. false. | Definition B. false. |
| Term (Chap 12) When working conditions are unpleasant or unsafe, worker productivity is affected. A. true B. false. | Definition A. true. |
| Term (Chap 12) Automated equipment requires consistently uniform components to function efficiently. A. true B. false. | Definition A. true. |
| Term (Chap 12) Most automated equipment is dedicated, or designed to perform one function. A. true B. false. | Definition A. true. |
| Term (Chap 12) A matrix begins by stating the problem and proposing a solution supported by rationale. A. true B. false. | Definition B. false. |
| Term (Chap 12) The time required to recover the cost of new equipment, either through savings in labor or materials, is called __________.. | Definition payback period. |
| Term (Chap 12) The money a business spends to purchase robots is a __________ investment.. | Definition capital. |
| Term (Chap 12) Investments made in equipment or other business/production assets which are projected to prevent additional costs in the future are __________.. | Definition avoidance costs. |
| Term (Chap 12) The key elements of various robots can be compared by constructing a(n) __________.. | Definition matrix. |
| Term (Chap 12) Proposals should always contain __________ justification.. | Definition economic. |
| Term (Chap 12) Describe the steps in developing a successful implementation plan.. | Definition |